# FPP Change Form

Change Request Number: 10LGO0001

Date: October 26, 2010

Proposed by: Greg Moody

Proposed Change:

Update operational language in the FPP describing the operation change from low crest to high crest surface spillbay weir.

Section 2.3.1.2. Fish Passage Period (April 1 through December 15)

g. Surface Spillbay Weir (SSW)

- Spring fish passage season will start with SSW-LO in place, even if river discharges are less than 75 kcfs.

- After the spring freshet has passed, the SSW-HI will be installed after river discharge falls below 75 kcfs for three days in a row and will remain in place for the rest of the season, even if river discharges subsequently increase above 75 kcfs.  The trigger to change to high crest is further based on the following:

    - forecasts indicate continuing lower river discharges;

    - a review of the juvenile fish counts at Little Goose monitoring to prevent changes during the middle of peak outmigration;

    - coordinated with regional fish managers.

- The uniform spill pattern, with no SSW operating, will be used when river discharges are too low to maintain a 30% spill ratio with a SW operating, 
or as an alternate pattern when the SW must be closed for any reason, such as when switching from SW-LO to SW-HI.

- When summer discharge falls below 35kcfs, and coordinated through CENWW-OD-T, the SSW will be removed from service within three regular workdays.

Reason for Change:

First year of operation of the SSW at Little Goose had very basic information on the operation of the SSW in the footnotes to the spill tables.  This brings the operating information from the footnotes into the main text of the FFP.

Comments from others:

Record of Final Action: