# FPP Change Form

Change Request Number: 10LWG001 Summer Spill Patterns

Date:	July 13, 2009

Proposed by: Tim Wik (USACE – Walla Walla)

John Bailey (USACE – Walla Walla)

Proposed Change:

Add Table LWG-12 to the Lower Granite section of the fish passage plan.  This table is to be used only for summer spill period.  There is no change in spill volume, only a change in spill pattern is proposed.  See attachment for proposed pattern.

Reason for Change:

Summer spill patterns for Lower Granite Dam were discussed at the late-April FFDRWG meeting.  Previous testing seemed to indicate higher survival with a "bulk" pattern (one bay open 4 stops, along with RSW) than with the standard flat pattern.  At the FFDRWG meeting, NOAA, BPA and CRITFC representatives agreed that a bulk pattern should be adopted for summer spill operations.

Comments from others:

-----Original Message-----

From: Kiefer,Russell [mailto:russ.kiefer@idfg.idaho.gov]

Sent: Monday, July 13, 2009 2:30 PM

To: Bailey, John C NWW

Subject: RE: FPOM coordination: New summer spill pattern at Lower Granite

John,

IDFG is in concurrence.

-----Original Message-----

From: BILL HEVLIN [mailto:Bill.Hevlin@noaa.gov]

Sent: Monday, July 13, 2009 3:42 PM

To: Bailey, John C NWW

Cc: Paul Wagner

Subject: Re: FPOM coordination: New summer spill pattern at Lower Granite

John,

Looks good, thanks for changing this.

Bill Hevlin

Record of Final Action:

## Table LWG-12.  Lower Granite summer spillway pattern for fish passage (with RSW operating at pool elevation 734).

| Spill Bay |  |  |  |  |  |  |  | Total Stops | Total Spill |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |
| 3.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3.5 | 6.1 |
| 3.5 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 4.5 | 7.9 |
| 3.5 | 0 | 2 | 0 | 0 | 0 | 0 | 0 | 5.5 | 9.6 |
| 3.5 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 6.5 | 11.4 |
| 3.5 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 7.5 | 13.1 |
| 3.5 | 0 | 4 | 0 | 0 | 0 | 1 | 0 | 8.5 | 14.9 |
| 3.5 | 0 | 4 | 0 | 0 | 1 | 1 | 0 | 9.5 | 16.6 |
| 3.5 | 0 | 4 | 0 | 0 | 1 | 1 | 1 | 10.5 | 18.4 |
| 3.5 | 0 | 4 | 0 | 1 | 1 | 1 | 1 | 11.5 | 20.1 |
| 3.5 | 1 | 4 | 0 | 1 | 1 | 1 | 1 | 12.5 | 21.9 |
| 3.5 | 1 | 4 | 1 | 1 | 1 | 1 | 1 | 13.5 | 23.6 |
| 3.5 | 1 | 5 | 1 | 1 | 1 | 1 | 1 | 14.5 | 25.4 |
| 3.5 | 1 | 5 | 1 | 2 | 1 | 1 | 1 | 15.5 | 27.1 |
| 3.5 | 1 | 5 | 1 | 3 | 1 | 1 | 1 | 16.5 | 28.9 |
| 3.5 | 1 | 5 | 1 | 4 | 1 | 1 | 1 | 17.5 | 30.6 |
| 3.5 | 1 | 5 | 1 | 5 | 1 | 1 | 1 | 18.5 | 32.4 |
| 3.5 | 1 | 5 | 1 | 5 | 1 | 2 | 1 | 19.5 | 34.1 |
| 3.5 | 1 | 5 | 1 | 5 | 1 | 3 | 1 | 20.5 | 35.9 |
| 3.5 | 1 | 5 | 1 | 5 | 1 | 4 | 1 | 21.5 | 37.6 |
| 3.5 | 1 | 5 | 1 | 5 | 1 | 5 | 1 | 22.5 | 39.4 |
| 3.5 | 1 | 5 | 2 | 5 | 1 | 5 | 1 | 23.5 | 41.1 |
| 3.5 | 1 | 5 | 3 | 5 | 1 | 5 | 1 | 24.5 | 42.9 |
| 3.5 | 1 | 5 | 4 | 5 | 1 | 5 | 1 | 25.5 | 44.6 |
| 3.5 | 1 | 5 | 5 | 5 | 1 | 5 | 1 | 26.5 | 46.4 |
| 3.5 | 1 | 5 | 5 | 5 | 2 | 5 | 1 | 27.5 | 48.1 |
| 3.5 | 1 | 5 | 5 | 5 | 3 | 5 | 1 | 28.5 | 49.9 |
| 3.5 | 1 | 5 | 5 | 5 | 4 | 5 | 1 | 29.5 | 51.6 |
| 3.5 | 1 | 5 | 5 | 5 | 5 | 5 | 1 | 30.5 | 53.4 |
| 3.5 | 1 | 5 | 5 | 5 | 5 | 5 | 2 | 31.5 | 55.1 |
| 3.5 | 1 | 5 | 5 | 5 | 5 | 5 | 3 | 32.5 | 56.9 |
| 3.5 | 1 | 5 | 5 | 5 | 5 | 5 | 4 | 33.5 | 58.6 |
| 3.5 | 1 | 5 | 5 | 5 | 5 | 5 | 5 | 34.5 | 60.4 |
| 3.5 | 2 | 5 | 5 | 5 | 5 | 5 | 5 | 35.5 | 62.1 |
| 3.5 | 3 | 5 | 5 | 5 | 5 | 5 | 5 | 36.5 | 63.9 |
| 3.5 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 37.5 | 65.6 |
| 3.5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 38.5 | 67.4 |
| 3.5 | 5 | 5 | 6 | 5 | 5 | 5 | 5 | 39.5 | 69.1 |
| 3.5 | 5 | 5 | 6 | 6 | 5 | 5 | 5 | 40.5 | 70.9 |
| 3.5 | 5 | 6 | 6 | 6 | 5 | 5 | 5 | 41.5 | 72.6 |
| 3.5 | 5 | 6 | 6 | 6 | 6 | 5 | 5 | 42.5 | 74.4 |
| 3.5 | 5 | 6 | 6 | 6 | 6 | 6 | 5 | 43.5 | 76.1 |

## Table LWG-12 (continued).  Lower Granite summer spillway pattern for fish passage (with RSW operating at pool elevation 734).

| Spill Bay |  |  |  |  |  |  |  | Total Stops | Total Spill |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |
| 3.5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 44.5 | 77.9 |
| 3.5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 45.5 | 79.6 |
| 3.5 | 6 | 6 | 7 | 6 | 6 | 6 | 6 | 46.5 | 81.4 |
| 3.5 | 6 | 6 | 7 | 7 | 6 | 6 | 6 | 47.5 | 83.1 |
| 3.5 | 6 | 7 | 7 | 7 | 6 | 6 | 6 | 48.5 | 84.9 |
| 3.5 | 6 | 7 | 7 | 7 | 7 | 6 | 6 | 49.5 | 86.6 |
| 3.5 | 6 | 7 | 7 | 7 | 7 | 7 | 6 | 50.5 | 88.4 |
| 3.5 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 51.5 | 90.1 |
| 3.5 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 52.5 | 91.9 |
| 3.5 | 7 | 7 | 8 | 7 | 7 | 7 | 7 | 53.5 | 93.6 |

Note:  At approximately 3.5 stops, the tainter gate no longer regulates flow through the RSW.  The tainter gate should be raised at least 9 stops so the gate does not interfere with the spillbay flow.Note:  Spillbay discharge at pool elevation 734:

Stops           Discharge (kcfs) (without RSW)		Stops           Discharge (kcfs) (without RSW)

1		1.7					5		  9.1

2		3.5					6		11.0

3		5.4					7		12.8

4		7.2					8		14.7

Discharge (kcfs) (with RSW)-- RSW 3.5 stops or more  6.7