January 14, 2010

FILE MEMORANDUM

FROM:            Thomas K Lorz

SUBJECT:  	Delayed Installation of ESBS’s at McNary Dam in 2010

Background :

Before the 2009 passage season, at the request of CRITFC and its member tribes, the region was asked to consider a delayed installation of guidance screens (ESBS) at McNary Dam.  Normally, these screens would be deployed per the Fish Passage Plan (FPP) guidance on or before April 1.  Given the extremely small numbers of adult lamprey passing McNary Dam in recent years, delayed screen installation was requested as a potential means to reduce loss of juvenile lamprey during some of the last good juvenile outmigration years.  The Corps  implemented the request in 2009 noting that juvenile lamprey appear to have an early peak in outmigration a week or so before the spring juvenile salmon migration begins to sharply rise.  The weight of existing evidence suggests that leaving the screens out for the first week or two of April could improve lamprey survival (which are known to suffer significant mortality on the intake screen systems) while not impacting too many salmon smolts.  While there is evidence that juvenile lamprey pass through the bottom of turbine intake screens, several studies indicate that juvenile lamprey may be uniformly distributed as they enter turbine intakes.  As stated in the Tribal Lamprey Restoration Plan (CRITFC 2008):

For many years the common assumption was that most juvenile lamprey travel along the bottom of the river during their approach to dams and pass through turbine intakes under turbine intake screens. Among other things, this conclusion arose from the fact that juvenile lamprey lack a swim bladder, in contrast to juvenile salmon.  However, in several cases, data from lamprey trapped on fyke nets placed in turbines behind turbine intake screens and on the screens themselves indicate that they travel higher in the vertical water column than previously believed. For example, fyke net tests at Priest Rapids Dam indicate that juvenile lamprey were found nearly equally distributed from the top to the bottom of the turbine gatewell slot (Carlson 1995 unpublished data).  Moursund et al. (2003) found that 86% of juvenile lamprey found on the John Day Dam extended length turbine screens were within the top 10% and bottom 10% of the screen face (Figure 18).  They also documented 91% of PIT-tagged lamprey and 14% of run-of-river lamprey were captured at fyke net levels 1-4 behind the turbine intake screen.

Figure 1.  Juvenile salmon and lamprey vertical and horizontal distribution on an ESBS as denoted by underwater cameras. Gray areas not sampled. Units in feet (Moursund et al. 2003 in CRITFC 2008).

Regardless of the exact number of juvenile lamprey that are impinged by turbine screens, the existing evidence indicates that screens are a considerable problem for juvenile lamprey that needs to be addressed.

Figure 2.  Juvenile lamprey run timing at McNary Dam from 1994-2005 (From CRITFC 2008- Bleich and Moursund 2006 based on Smolt Monitoring Program data).

Existing data indicates that the peak of the juvenile lamprey migration over many years is roughly in the middle of May (Figure 2).  The Corps conducted underwater camera observations of extended length screens at McNary Dam and conducted impingement studies at John Day.  While impingement of lamprey on the screens was observed, problems with the camera prevented a reliable accounting of the impact.  Subsequently, the Corps funded studies at McNary and John Day to further address the issue (Moursund et al. 2000; Moursund et al. 2002; Moursund et al. 2003; Bleich and Moursund 2006).  Research indicated that 98% of impinged lamprey were unable to free themselves from the screens at typical screen face velocities (Moursund et al. 2000). As evidence of impingement persisted, recommendations were made to mitigate these impacts by reducing screen gap size from 3.175 mm to 1.75 mm (Bleich and Moursund 2006).  However, the cost of replacing screens with reduced gap size is expensive and has limited testing.  Cost estimates and more evaluations would need to be done before full scale installation could begin.

The Corps’ McNary operation for 2009 included delayed installation of the screens until April 7, with the installation completed by April 14.  The Corps was also tasked with putting together a change form for the 2010 FPP that would provide for future delayed installation of these screens at McNary.

The new language in the 2010 FPP reads as follows:

Section 2.3.1.2.b.1.  Operate ESBSs with flow vanes attached to the screen.  Installation of the ESBSs will not start before the first Monday of April and will be completed within the following two weeks.

Gary Fredricks from NOAA prepared an analysis describing best estimates for the impacts to early migrating juvenile salmonids.  I was tasked to do the same for juvenile lamprey.  I assumed that all the screens would be installed by April 19th, the latest installation date possible under this language.  Further I assumed that units would have screens installed in reverse order of prioritization.  At issue is the lack of data for juvenile lamprey passage at the projects.  What information that was available was utilized for this analysis.

Analysis:

I consulted with members from the Fish Passage Center and they produced the SMP daily collection index of lamprey passing McNary dam from April 1 to April 30 across the years of 2000 to 2009 (See table 1).  It is important to realize that these data only reflect macropthalmia and not ammocetes because ammocetes are rarely found in SMP sampling.  Furthermore, these numbers do not account for juveniles that go through screens or are lost through the gap between the top of the screens and the gatewell intake.  Thus, these numbers are likely very conservative and may not be representative of the actual juvenile lamprey passage for any particular year.

Table 1: Daily Collection Numbers of Juvenile Lamprey at McNary expanded for a 24 hour period

| Date | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-Apr | 360 |  |  |  |  |  |  |  |  |  | 36 |
| 2-Apr | 408 |  |  |  |  |  |  |  |  |  | 41 |
| 3-Apr | 594 | 68 | 10 | 25 | 40 | 230 | 80 | 20 | 30 |  | 110 |
| 4-Apr | 502 | 76 | 636 | 17 | 60 | 157 | 94 | 0 | 45 |  | 159 |
| 5-Apr | 600 | 144 | 1155 | 7 | 70 | 120 | 100 | 0 | 60 |  | 226 |
| 6-Apr | 612 | 60 | 308 | 371 | 63 | 108 | 95 | 0 | 45 |  | 166 |
| 7-Apr | 240 | 40 | 300 | 707 | 55 | 95 | 90 | 280 | 30 |  | 184 |
| 8-Apr | 1938 | 48 | 128 | 560 | 52 | 113 | 168 | 210 | 82 |  | 330 |
| 9-Apr | 1218 | 72 | 450 | 400 | 50 | 130 | 245 | 140 | 600 | 10 | 332 |
| 10-Apr | 600 | 120 | 284 | 436 | 38 | 95 | 156 | 78 | 200 | 11 | 202 |
| 11-Apr | 1388 | 68 | 316 | 468 | 25 | 63 | 71 | 50 | 120 | 12 | 258 |
| 12-Apr | 1680 | 76 | 250 | 358 | 29 | 78 | 103 | 32 | 67 | 106 | 278 |
| 13-Apr | 960 | 61 | 276 | 233 | 33 | 95 | 170 | 6 | 40 | 232 | 211 |
| 14-Apr | 311 | 116 | 232 | 350 | 36 | 94 | 235 | 173 | 16 | 315 | 188 |
| 15-Apr | 240 | 96 | 199 | 513 | 40 | 91 | 298 | 305 | 0 | 420 | 220 |
| 16-Apr | 280 | 148 | 342 | 275 | 20 | 68 | 427 | 246 | 3 | 342 | 215 |
| 17-Apr | 170 | 331 | 1570 | 20 | 86 | 40 | 679 | 107 | 5 | 270 | 328 |
| 18-Apr | 502 | 200 | 860 | 74 | 69 | 35 | 2776 | 113 | 9 | 140 | 478 |
| 19-Apr | 650 | 76 | 2610 | 125 | 40 | 33 | 13905 | 120 | 13 | 0 | 1757 |
| 20-Apr | 180 | 17 | 5136 | 189 | 79 | 39 | 8895 | 50 | 10 | 176 | 1477 |
| 21-Apr | 123 | 48 | 6101 | 320 | 127 | 45 | 4160 | 9 | 6 | 344 | 1128 |
| 22-Apr | 160 | 24 | 5100 | 240 | 101 | 63 | 3506 | 9 | 18 | 176 | 940 |
| 23-Apr | 210 | 96 | 6088 | 160 | 28 | 80 | 2811 | 10 | 29 | 0 | 951 |
| 24-Apr | 920 | 23 | 3300 | 146 | 4 | 58 | 2140 | 39 | 23 | 70 | 672 |
| 25-Apr | 470 | 80 | 2800 | 133 | 30 | 34 | 1536 | 100 | 17 | 337 | 554 |
| 26-Apr | 2380 | 120 | 7587 | 275 | 0 | 32 | 1095 | 0 | 22 | 484 | 1200 |
| 27-Apr | 1680 |  | 3200 | 650 | 0 | 30 | 851 | 0 | 28 | 753 | 799 |
| 28-Apr | 540 |  | 2700 | 670 | 0 | 35 | 753 | 0 | 32 | 822 | 617 |
| 29-Apr | 676 | 30 | 3842 | 704 | 201 | 39 | 661 | 30 | 37 | 900 | 712 |
| 30-Apr | 950 | 15 | 2140 | 562 | 0 | 22 | 566 | 339 | 58 | 633 | 529 |

With the collection information I then computed a survival estimate at McNary dam with and without screens prior to spill and then post spill. I assumed a 1:1 ratio for lamprey to flow for a particular passage route, i.e. 40% spill would equate to 40% of the lamprey using the spillway.  For the survival estimates I computed a low end FGE value and a high FGE to get a range of potential impacts.  I then expanded the lamprey collection numbers to a population and estimated the number of juvenile lamprey that would be impacted by delaying the screen installation until April 19.

Assumptions for FGE and Survival:

Due to the lack of significant research specifically dealing with lamprey, most of the information had to be derived.  Lamprey Fish Guidance (FGE) has been reported at approximately25% at Bonneville 2nd Powerhouse (Monk et. al, 2002) and 22% at John Day Dam (Moursund et al. 2003).  Based on this and the information noted above that lamprey use the entire water column and FGES may therefore be higher, I analyzed both 25% and 50% FGE for McNary to better ensure the likely impact was captured.  Survival numbers are based in part on salmonid research and best professional judgment.

Fish Guidance:  	Low 25%          	High 50%

Mortality:				Low 			High

Screens:			25%			60%

Turbine:			10%			20%

Spill:				0%			5%

Using the above estimates for guidance and survival I was able to estimate mortality for the different operational scenarios.  These are outlined in Table 2.

Table 2: Percent Mortality & Difference Between Low and High End of Range

|  |  | Percent Mortality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mortality w/screen | w/o Spill |  | W/Spill |  | Difference |
| FGE - .25 Low Mort | 13.75 |  | 8.25 |  | 5.5 |
| FGE - .50 | 40 |  | 26 |  | 14 |
| FGE - .25 High Mort | 30 |  | 20 |  | 10 |
| Mortality w/o screen |  |  |  |  |  |
| FGE - .25 | 10 |  | 6 |  | 4 |
| FGE - .50 | 20 |  | 14 |  | 6 |
| Range w/wo Screens | 3.75 |  | 2.25 |  |  |
|  | 20 |  | 12 |  |  |

With the population and the above mortality estimates I then went through several operational scenarios and estimated the number of mortalities and percent of the April population that this represented.  I had previously consulted with FPC to estimate the passage season for juvenile lamprey through the use of SMP data.  Refer to Figure 3 and table 3.  (Note, week 14 is corresponds to last week of March and the first week of April depending on the days of the week for that year)

Figure 3: Percent Passage of Juvenile Lamprey at McNary Dam

Table 3. Percent weekly average Juvenile Lamprey passage timing at McNary Dam based on average weekly passage index for the years 2000 to 2008.

| Week at McNary Dam | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | Overall |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 0.4 | 9.7 | 7.9 | 15.1 | 0.2 | 0.0 | 3.3 | 2.4 | 0.2 | 4.4 |
| 15 | 20.2 | 14.5 | 9.3 | 17.6 | 3.8 | 4.6 | 6.9 | 10.0 | 0.5 | 9.7 |
| 16 | 12.8 | 22.7 | 12.9 | 4.6 | 2.7 | 3.3 | 16.8 | 9.2 | 0.2 | 9.5 |
| 17 | 4.9 | 3.2 | 13.0 | 1.8 | 2.9 | 1.7 | 4.6 | 1.5 | 0.3 | 3.8 |
| 18 | 7.1 | 1.1 | 8.6 | 1.3 | 1.0 | 1.7 | 4.2 | 2.2 | 0.7 | 3.1 |
| 19 | 3.5 | 2.1 | 1.4 | 0.7 | 1.0 | 0.3 | 3.3 | 0.5 | 1.3 | 1.6 |
| 20 | 2.0 | 3.5 | 0.1 | 0.6 | 0.8 | 0.9 | 5.4 | 1.3 | 1.0 | 1.7 |
| 21 | 2.5 | 8.7 | 0.3 | 0.5 | 1.5 | 6.5 | 8.4 | 5.7 | 28.5 | 6.9 |
| 22 | 2.9 | 3.4 | 1.4 | 0.6 | 2.4 | 30.6 | 17.6 | 1.8 | 32.5 | 10.4 |
| 23 | 5.4 | 9.2 | 17.6 | 33.7 | 32.1 | 21.2 | 12.8 | 6.7 | 20.7 | 17.7 |
| 24 | 10.7 | 5.4 | 6.1 | 18.2 | 27.5 | 5.7 | 3.8 | 11.6 | 2.6 | 10.2 |
| 25 | 3.8 | 4.4 | 2.2 | 0.7 | 9.1 | 3.2 | 2.1 | 10.3 | 1.7 | 4.2 |
| 26 | 2.8 | 1.1 | 1.9 | 0.7 | 3.6 | 1.2 | 1.6 | 6.1 | 2.9 | 2.4 |
| 27 | 1.7 | 0.4 | 4.0 | 0.7 | 0.4 | 0.6 | 1.3 | 2.6 | 0.4 | 1.3 |
| 28 | 1.7 | 1.0 | 2.7 | 0.3 | 0.8 | 0.8 | 1.2 | 5.7 | 0.7 | 1.7 |
| 29 | 1.3 | 1.8 | 1.8 | 0.3 | 1.3 | 1.6 | 1.1 | 6.2 | 0.8 | 1.8 |
| 30 | 1.1 | 0.1 | 0.9 | 0.2 | 1.0 | 3.7 | 0.7 | 1.6 | 0.7 | 1.1 |
| 31 | 0.8 | 0.5 | 1.8 | 0.3 | 0.8 | 3.3 | 1.1 | 1.6 | 0.6 | 1.2 |
| 32 | 0.2 | 0.0 | 3.9 | 0.2 | 0.8 | 3.2 | 0.4 | 1.3 | 0.8 | 1.2 |
| 33 | 0.3 | 0.2 | 0.3 | 0.2 | 1.1 | 2.1 | 1.1 | 3.9 | 0.5 | 1.1 |
| 34 | 0.6 | 0.2 | 0.1 | 0.3 | 0.7 | 1.2 | 1.0 | 3.3 | 0.6 | 0.9 |
| 35 | 0.7 | 0.1 | 0.1 | 0.5 | 1.9 | 0.5 | 0.6 | 1.6 | 1.1 | 0.8 |
| 36 | 0.6 | 0.5 | 0.4 | 0.4 | 0.8 | 1.2 | 0.6 | 1.1 | 0.3 | 0.7 |
| 37 | 0.6 | 0.2 | 0.2 | 0.3 | 1.0 | 0.6 | 0.0 | 1.8 | 0.1 | 0.5 |
| 38 | 1.0 | 0.4 | 0.1 | 0.3 | 1.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.3 |
| 39 | 0.4 | 0.7 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |
| 40 | 1.3 | 0.7 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |
| 41 | 0.9 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 42 | 1.2 | 0.6 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |
| 43 | 0.1 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 44 | 0.2 | 0.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 45 | 0.5 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 46 | 0.8 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 47 | 1.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 48 | 2.1 | 2.7 | 0.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 |
| 49 | 1.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |

From the above graphs it appears ~20 -25% of SMP juvenile lamprey may be passing the project during the Corps’ proposed April delayed installation of the screens.

Results:

Table 4 depicts juvenile lamprey mortality differences between the proposed delayed installation and the FPP scheduled installation operations.  The level of improvements that could be achieved for juvenile lamprey by delaying screen installation are illustrated in the column labeled difference.  The actual level of improvement is likely somewhere in-between.  FGE and screen mortality are the most important factors in estimating the numbers of lamprey impacted and there is little information for either.  The 25% guidance was based on fyke net studies done at lower river projects specifically, Bonneville and some work at John Day.  Bonneville Powerhouse 2 is equipped with Standard Length Screens, STS, and while McNary has extended screens which will likely increase guidance/impingement an unknown amount.  John Day has extended length screens but has a deeper forebay than McNary which will likely impact distribution of lamprey throughout the water column.  As for mortality, there is anecdotal information mostly from video and instances when screen cleaners break so the impinged juvenile lamprey are not removed and can be enumerated.  This is why I included a relative wide range for mortality.

Table 4:  Estimates for number and percent of April outmigration impacted by different scenarios at McNary dam.

|  | Current Installation Schedule |  | Delayed Installation |  | Difference |
| --- | --- | --- | --- | --- | --- |
| Lamprey Mortality (no.) |  |  |  |  |  |
| High FGE Estimate | 4,849 |  | 2,562 |  | 2,287 |
| Low FGE estimate | 3,144 |  | 2,287 |  | 857 |
| Percentage of April Population |  |  |  |  |  |
| High FGE estimate | 6.4% |  | 3.4% |  | 3.0% |
| Low FGE Estimate | 2.1% |  | 1.5% |  | 0.6% |

Based only upon the limited data on macropthlamia sampled at the McNary SMP sites, the likely range of impact is somewhere between ~2300 to 900 juvenile lamprey which equates to 3.5 to 0.6% of the lamprey population at McNary in April.  For the entire juvenile lamprey population, including ammocetes that passes through McNary the percentages of loss are likely higher.

Conclusion

Based upon this analysis and the weight of existing scientific evidence, a juvenile lamprey survival benefit could be found by delaying the installation of extended screens at McNary Dam.  Given the extremely low numbers of juvenile and adult lamprey upstream of McNary Dam, and a real potential for extinction that lamprey are facing, modifying operations that can benefit lamprey and pose limited impact to salmonids should be considered.  This analysis assumed that screens would be installed by April 19th.  There are several key uncertainties that affect this analysis:

- FGE is one of the primary drivers for this analysis.  There is conflicting data about the level of guidance for the different life histories of juvenile lamprey.

- The SMP collection numbers which determine the population passing McNary only include fish that were captured in the facility; impinged juveniles that either went through or were sweep off through the gap would not be accounted for.  This should not affect the percentages but would increase the number lamprey mortalities.  Thus, the mortality rate of 60% for the bypass passed lamprey could be low.

If the screens are installed earlier than the 19th the survival benefit would decrease.

The analysis also indicates that almost the same level of survival improvement is achieved when spill was initiated on April 10th.  This suggests another possible operation: initiate some level of spill prior to April 10th to reduce lamprey passage at the powerhouse and increase the spill level on April 10th above the current 40% of river flow until the screens are installed.  This option would also reduce the impacts to salmonid migrants and better protect life history diversity necessary to recover both salmon and lamprey.

References

CRITFC.  Tribal Pacific Lamprey Restoration Plan for the Columbia River Basin. Draft.  Portland, OR.

Fryer, J.  2002 Annual variations in timing, duration and success of seaward migration by sockeye salmon (Oncorhynchus nerka) smolts originating from the Okanagan and Wenachee subbasins.  CRITFC unpublished report.  Portland, OR.

Monk et al., 2002 Evaluation of Turbine intake Modifications at the Bonneville Second Powerhouse, 2002.  For Corps of Engineers, Portland District, NOAA fisheries, Pasco, WA.  (http://www.nwp.uasace.army.mil/pm/e/reports/afep/bon/BON2 Turb 02.pdf)

Mousund,R.A., D.D. Dauble and M.D. Bleich.  2000.  Effects of John Day Dam bypass screens and project operations on the behavior and survival of juvenile Pacific lamprey (Lampetra tridentata). For Corps of Engineers, Portland District.  By Pacific Northwest Laboratory, Richland, Washington.

Moursund, R.A., R.P. Mueller, T.M. Degerman, and D.D. Dauble. 2001. Effects of dam passage on juvenile Pacific lamprey Lampetra tridentata. Final Report. Prepared for U.S. Army Corps of Engineers, Portland District.

Moursund, R.A., M.K. Bleich, K.D. Ham, R.P. Mueller. 2002. Evaluation of the effects of extended length submerged bar screens on migrating juvenile Pacific lamprey (Lampetra tridentata) at John Day Dam in 2002. Prepared for the U.S. Army Corps of Engineers, Project No. DE-AC06-76RL01830.

Moursund, R. A, M. D. Bleich, K. D. Ham, and R. P. Mueller. 2003. Evaluation of the effects of extended length submerged bar screens on migrating juvenile Pacific lamprey at John Day Dam in 2002. Final Report of Research, U. S. Army Corps of Engineers.