OFFICIAL COORDINATION REQUEST FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 12LMN05 Lower Monumental summer spill change from 115 kcfs /120% TDG – 24 hours per day to 17 kcfs – 24 hours per day on June 9th or when flows allow.

COORDINATION DATE- 6 June 2012

PROJECT- Lower Monumental Lock and Dam

RESPONSE DATE- 7 June 2012, 3:00 PM

Description of the problem

The 2012 FOP states “Performance standard testing at the TDG Gas Cap (spring) and at 17 kcfs (summer) spill will occur in 2012 at Lower Monumental Dam.  Testing began in late April and will continue through July.  The dates of testing are dependent on the size and availability of fish for tagging.  Final dates for testing will be coordinated through the SRWG.  The “bulk” spill pattern as described in FPP Section 7 will be used.  Based on previous years’ study results, dam survival is higher using the “bulk” spill pattern compared to the “uniform” spill pattern”.

The first release of JSATS tagged fish has already occurred.  The start date of summer spill would be moved up from June 20 to June 9 or as soon as flows allow.

Type of outage required

No outage.  Just a change from spring to summer spill operations as permitted by river flows.

Impact on facility operation

We propose switching to summer operations (17kcfs spill 24 hours) as flows come down to around 115 Kcfs as predicted on June 9th, or as soon as flows decrease to manageable levels.  The first releases of fish at LOMO began last Sunday and the numbers of subs in the collection have been low so far, and the release numbers as of yesterday were less than 500 fish total out of 11,000. Switching to summer spill sooner will allow for a better evaluation of summer conditions.

Dates of impacts/repairs

9 Jun 2012, or as soon as flows allow.

Length of time for repairs

NA

Expected impacts on fish passage

Lower Monumental SMP data shows subyearling Chinook making up approximately 73% of the daily collection at the fish facility as of 05 June 2012 as compared to 68.4% in 2011, 37.4% in 2010 and 77.6% in 2009 on this date.  Implementing summer spill operations at Lower Monumental on June 9th will better match the run and performance standard testing timing.

Comments from agencies

Final results

Thank you,

John Bailey