26 November 2008

To: David Clugston (USACE)

From: Matt Keefer and Chris Caudill

Re: Hourly detection distributions for adult salmon and steelhead at USACE dams

Introduction:  This letter report summarizes the diel passage behavior of adult salmonids at lower Columbia and Snake River dams.  The data were assembled to help managers identify the best times for fishway maintenance and other activities that potentially disrupt upstream adult salmonid passage.  Adult PIT-tag data and the extensive radiotelemetry studies conducted by the UI and NMFS (e.g., Keefer et al. 2003, 2007, 2008a, 2008b; Burke et al. 2005; Stuehrenberg et al. 2005) have shown that the majority of adult activity in fishways occurs during daylight hours.  The relatively limited night-time activity by salmonids has mostly been by fish that entered fishways late in the day.

Methods and Results:  Here we generated time-of-day distributions of adult fish activity at fishway entrances (approaches and entries) and top-of-ladder sites (exits into forebays).  These two areas bracket the fishway passage environment.  We used five years of telemetry data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead (2000-2004) and a single year for sockeye salmon (1997).  In these years, all adult fish were collected and radio-tagged at Bonneville Dam.  Sample sizes ranged from several hundred to more than 1,100 fish per run per year, and were smallest in 2004.

The level of telemetry coverage differed between dams and years.  However, top-of-ladder sites were continuously monitored at all eight dams in all years and most fishway entrances were also monitored at all dams.  The combination of large sample sizes and saturation antenna coverage resulted in large numbers of detections, particularly for fishway approaches (Table 1).

Time-of-day distributions are presented graphically in Figures 1-8.  Results were generally similar across runs and dams, with the vast majority of tagged fish detected during daylight hours at each site.  Distributions for fishway approaches typically showed a rapid increase in detections after dawn, often followed by a slight lull in mid-day and another increase in late afternoon.  Distributions for fishway entrances were very similar to those for approaches, but averaged slightly later than approaches.  Top-of-ladder distributions were later overall, reflecting the time fish used to pass through fishways and up ladders.  At some dams, peak top-of-ladder passage occurred in the afternoon.  At other sites, there was some evidence for two peak passage times, with a lull in mid-day.  Differences between runs were relatively limited, and were likely related more to day length or water temperature at different seasons than to biological differences.  It is possible that differences in swimming ability had an effect (e.g., the relatively late timing of sockeye salmon).

These results indicate that adult salmonid activity at dams was consistently lowest at night.  The hours between approximately 22:00 and 04:00 had the lowest fishway detection numbers (Figures 1-8).  At most dams, ≤ 5% of the detections for each run were between midnight and 04:00 (Figure 9).  This suggests that this time of night may be best time for maintenance or other actions, at least for adult salmonids.  We note that adult Pacific lamprey are primarily nocturnal, and have passage timing distributions that are nearly the opposite of those for salmonids (see Figure 10).

Literature cited

Burke, B.J., K.E. Frick, M.L. Moser, T.J. Bohn, and T. C. Bjornn. 2005. Adult fall Chinook salmon passage through fishways at lower Columbia River dams in 1998, 2000, and 2001.  Report of National Marine Fisheries Service to U.S. Army Corps of Engineers, Portland district.

Keefer, M.L, T.C. Bjornn, C.A. Peery, K.R. Tolotti, R.R. Ringe, P.J. Keniry, and L.C. Stuehrenberg.  2003. Adult spring and summer Chinook salmon passage through fishways and transition pools at Bonneville, McNary, Ice Harbor, and Lower Granite dams in 1996.  Technical report 2003-5 of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

Keefer, M. L., C. A. Peery, T. C. Bjornn, M. A. Jepson, K.R. Tolotti, R.R. Ringe, and L. C. Stuehrenberg.  2007.  Adult salmon and steelhead passage through fishways and transition pools at The Dalles Dam, 1997-2001.  Technical Report 2007-2 of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

Keefer, M. L., C. A. Peery, T. C. Bjornn, M. A. Jepson, K.R. Tolotti, S. R. Lee, and L. C. Stuehrenberg.  2008a.  Adult salmon and steelhead passage through fishways and transition pools at John Day Dam, 1997-2001.  Technical Report 2008-4 of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

Keefer, M. L., D. C. Joosten, C. L. Williams, C. M. Nauman, M. A. Jepson, C. A. Peery, T. C. Bjornn, R. R. Ringe, K.R. Tolotti, S. R. Lee, L. C. Stuehrenberg, M. M. Moser, and B. J. Burke.  2008b.  Adult salmon and steelhead passage through fishways and transition pools at Bonneville Dam, 1997-2002.  Technical Report 2008-5 of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

Keefer, M.L., C.T. Boggs, C.A. Peery, and M.L. Moser. In review.  Adult Pacific lamprey migration in the lower Columbia River: 2007 radiotelemetry and half duplex PIT-tag studies.  Technical Report of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

Stuehrenberg, L. C., M. L. Keefer, C. A. Peery, K. R. Tolotti, R. R. Ringe, T. C. Bjornn, and B. Burke. 2005.  Adult steelhead passage through fishways and transition pool at Bonneville, McNary, and Lower Granite dams - 1996.  Technical Report 2005-6 of Idaho Cooperative Fish and Wildlife Research Unit to U.S. Army Corps of Engineers, Portland and Walla Walla districts.

------------------------------------------------------------------------------------------------------------

Table 1.  Total numbers of fishway approaches, entries, and ladder exits recorded for radio-tagged adult salmonids at lower Columbia and Snake River dams.  Spring–summer Chinook salmon, fall Chinook salmon, and steelhead numbers are totals for 2000-2004 migration years; sockeye numbers are for 1997.

|  |  | Sp–Su | Fall |  |  |
| --- | --- | --- | --- | --- | --- |
| Behavior | Dam | Chinook | Chinook | Steelhead | Sockeye |
| Fishway approach | BON | 64,446 | 51,202 | 47,763 | 6,685 |
|  | TDA | 27,499 | 17,165 | 18,800 | 727 |
|  | JDA | 182,927 | 88,283 | 63,884 | 2,249 |
|  | MCN | 35,925 | 15,482 | 18,644 | 1,957 |
|  | IHA | 15,619 | 1,778 | 8,204 | - |
|  | LMN | 34,996 | 3,608 | 16,027 | - |
|  | LGO | 42,616 | 2,246 | 16,191 | - |
|  | LGR | 45,744 | 3,044 | 21,444 | - |
| Fishway entry | BON | 9,353 | 8,571 | 9,706 | 1,115 |
|  | TDA | 8,983 | 9,066 | 9,733 | 526 |
|  | JDA | 24,891 | 22,832 | 18,559 | 1,201 |
|  | MCN | 8,185 | 5,921 | 5,058 | 676 |
|  | IHA | 3,007 | 735 | 2,549 | - |
|  | LMN | 4,848 | 1,417 | 4,641 | - |
|  | LGO | 3,782 | 891 | 4,557 | - |
|  | LGR | 4,177 | 563 | 3,128 | - |
| Ladder exit | BON | 4,629 | 3,122 | 3,547 | 630 |
|  | TDA | 4,294 | 3,151 | 3,743 | 505 |
|  | JDA | 3,616 | 2,379 | 3066 | 446 |
|  | MCN | 3,272 | 1,902 | 2,696 | 196 |
|  | IHA | 1,733 | 257 | 1,890 | - |
|  | LMN | 1,552 | 238 | 1,939 | - |
|  | LGO | 1,662 | 217 | 1,812 | - |
|  | LGR | 1,680 | 180 | 1,686 | - |

Figure 1.  Time-of-day detection distributions for adult salmonids at Bonneville Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 2.  Time-of-day detection distributions for adult salmonids at The Dalles Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 3.  Time-of-day detection distributions for adult salmonids at John Day Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

.

Figure 4.  Time-of-day detection distributions for adult salmonids at McNary Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 5. Time-of-day detection distributions for adult salmonids at Ice Harbor Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 6. Time-of-day detection distributions for adult salmonids at Lower Monumental Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 7.  Time-of-day detection distributions for adult salmonids at Little Goose Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 8. Time-of-day detection distributions for adult salmonids at Lower Granite Dam, including 5 years of data for spring–summer Chinook salmon, fall Chinook salmon, and steelhead, and one year for sockeye salmon.

Figure 9.  Percentages of each run detected between 0–4 h (gray bars) and 20-24 h (black bars), by site, run, and dam.  Dams from left to right: BON, TDA, JDA, MCN, IHA, LMN, LGO, LGR.

Figure 10. Distributions of the times that HD PIT-tagged adult Pacific lamprey were detected passing top-of-ladder sites at Bonneville, The Dalles, John Day, and McNary dams in 2007.  (From Keefer et al. in review).