# FPP Change Request Form

Change Request Number: 14AppG003 BON AFF picket lead operation

Date Submitted:  17 September 2013

Project: BON

Requester Name, Agency: BON Project Fisheries

Location of Change:  Appendix G section 3 and section 4

Proposed Change:

### 3. Trapping Protocols - Fish Ladder Water Temperatures <70F

#### 3.9. Four picket leads will be allowed during trap operations for up to 4 hours.  After all picketed leads are raised, fish already in the AFF can be sampled for an additional one hour.  The picketed lead operations are as follows.   (*All counts are adult salmonids (including jacks) as enumerated at the Washington Shore count station for the previous day.  Assumes that 4 shad = 1 salmonid, e.g., 6000 salmonids + 4000 shad = 7000).

- zero – 6,000* - All 4 picket leads can be lowered for 4 continuous hours.

- 6,000 -12,000* - All 4 picket leads down for 3 hours.  At the 3rd hour raise at least one picket lead for ½ hour, and then continue sampling for an additional 1 hour.

- 12,000 -20,000* - All four picket leads down for 2 hours.  Three leads down for two hours..

- Greater than 20,000* - Two  leads down for four hours.

#### 3.10.  Researchers will also be required to monitor the ladder every hour to ensure that crowding is not taking place.  If evidence of crowding is occurring at least two picket leads will be raised.

- 4. Trapping Protocols - Fish Ladder Water Temperatures ≥70F

- 4.2. Between 70°F and 72°F, sampling will be permitted as defined below for up to four days per week from 0600-1030 hours to allow for U.S. v Oregon requirements and for nighttime lamprey trapping.  This operation will remain in effect until daily average water temperatures drop to ≤ 69.9F.  All sampling will cease when temperatures reach 72°F.  No sampling may resume until daily average water temperatures drop to ≤ 71.9°F.

- 4.2.1. Researchers may continue to work through fish in the holding pool for one hour after picket leads have been raised.

- 4.2.2. Project Fisheries will notify FPOM as soon as Weir 37 consistently violates FPP criteria.

- 4.2.3.The density criteria for picket lead operations will be altered and the operations will be as follows.  The density criteria and monitoring of the adult ladder by the researchers as outlined in 3.9.1 also apply.  (*All counts are adult salmonids (including jacks) as enumerated at the Washington Shore count station for the previous day.  Assumes that 4 shad = 1 salmonid, e.g., 6000 salmonids + 4000 shad = 7000).

- Zero – 3,000* - All four picket leads can be lowered for four continuous hours.

- 3,000-20,000* -  Three picket leads down for four hours.

- > 20,000* - Two  picket leads down for four hours.  All picket leads need to be raised by 10:30 am.

Justification for Change: This change is based on the ladder conditions of August/September 2013.  High fish numbers and high water temps may have led to large numbers of fish congregating in the bend below Weir 37.  Having all four leads down created a back up of fish.  The ½ with two leads up did not allow enough time for fish to move through the area and thin out.

When water temperatures are above 70F, it is even more important to allow fish to move through the fishway with as few impediments as possible.

Comments from others (Date – Name – Comments):

Record of Final Action (Date – Action):