# FPP Change Request Form

Change Request Number: 14BON009 2.5.4. Fish unit trash rack raking

Date Submitted:  17 September 2013

Project: BON

Requester Name, Agency: NWP

Location of Change:  BON 2.4.2.2.d and 2.5.4

Proposed Change:

2.5.4.d. Measure fish unit gatewell drawdown at least once per week.  When the head across trash racks exceeds 1.5', the trash racks will be cleaned that day.  This may be done by raking late in the workday.  However, if the head exceeds 3' or if the adult fishway head is reduced, the unit’s racks will be raked immediately, even if it is early in the day.

2.5.4.e. Take soundings annually at the PH2 Fish Unit intake and the BI exit/AWS intake to determine sediment accumulation and plan for the appropriate dredging need.  Fish Unit intake dredging is a key component to maintaining a reliable AWS system and should be prioritized during winter maintenance season.

Justification for Change: 2.4.2.2.d does not belong in the Juvenile Fish Passage Season section.  It needs to be moved to the Adult Fish Passage Season Powerhouse Two section.  Floating fish units should no longer be used as a primary debris removal method since the trashrake is effective at removing debris.  Floating the fish units is hard on the units, creates problems for lamprey at the entrances and in the collection channel, and violates FPP fishway criteria for head differential.  Trashrack raking should be the primary method for maintaining AWS reliability during fish passage season, along with FU intake dredging during winter maintenance season (when WS is OOS for maintenance).

2.5.1.2.k should also be moved to the Adult Fish Passage Season Powerhouse Two section.

Comments from others (Date – Name – Comments):

Record of Final Action (Date – Action):