# FPP Change Request Form

Change Request Number & Title:

Date Submitted:

Project: Lower Granite

Requester Name, Agency: Bill Hevlin, NOAA Fisheries

Location of Change - FPP Project and Section:

Specific alternative spill operations at Lower Granite Dam, different from the Fish Passage Plan, meant to reduce adult salmon tailrace delay and/or blockage by the improvement of tailrace hydraulic conditions conducive to fish locating fishway entrances and passage up the ladder.

Proposed Changes (in track changes to existing section):

Three Alternative Operations Are Proposed -

When a passage delay or blockage is noticed the project will alter spill operations, following the guidelines specified in alternative 1 below.  If adult passage is not noticeably stimulated with the 1st alternative operation, then the 2nd operation will be tried.  And if the 2nd alternative does not produce results, the 3rd operation will be used.

1st Alternative Operation:

At 5 am, close the RSW in spillbay one, and go to a uniform spill pattern with the same spill volume.  At 11 am, restore the RSW operation, with training spill.  Spill volume does not change during this alternative operation, just the spill pattern.  Unit one has priority operation, followed by units 2 and 3.

2nd Alternative Operation:

At 5 am, close the RSW, but continue training spill, and increase flow to the powerhouse south units at a volume similar to the RSW.  Unit one has priority operation, followed by units 2 and 3. At 11 am, restore the RSW operation with training spill.  The spill volume lost during the 6 hour period, due to the closure of the RSW, will be incrementally made up over the remaining 18 hours that day.

3rd Alternative Operation:

At 5 am, close all spillbays, including the RSW and training spill, and increase flow to the south powerhouse units at a similar volume.  Restore the RSW and training spill at 11 am, and incrementally over the remaining 18 hours make up the spill volume lost during the 6 hour period.

Trigger and Justification for Change:

The trigger/justification for the alternative spill operation will be any FPOM fishery agency representative raising concern about the possibility that an adult passage delay or blockage may be occurring at Lower Granite Dam.

Comments from others:

Record of Final Action: