# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	15BON005 – B-Branch Entrance Gates

Date Submitted:		October 9, 2014

Project:			BON

Requester Name, Agency:	Brian Bissell, BON Fisheries

Final Action:

FPP Section:  2.5.2.2. Adult Fish Passage Facilities – Main Dam Fish Ladders.

Justification for Change:  SO-SG-4N and -4S are located at the B-branch entrance. This entrance does not currently have a variable-width entrance weir and these sluice gates operate depending on tailwater conditions.

SW-SG-3N and -3S are located at the Cascades Island entrance. They are permanently closed due to the variable-width entrance weir installed in May 2009.

According to Steve Schlenker (CENWP-EC-HD) there is a hydraulic benefit to having the north sluice gate open prior to the south.

Proposed Change:

    - Main Dam Fish Ladders.

2.5.2.2. Ladder entrances at Cascades Island are labeled SW-SG (Washington sluice gate) and entrances at Bradford Island are labeled SO-SG (Oregon sluice gate).  Side entrances SW-SG-5 and SO-SG-7 shall remain closed.  Downstream entrances SW-SG-1 and SO-SG-2 shall operate as continuously-open, free-flowing vertical slots.  Downstream entrances SW-SG-3 and SO-SG-4, adjacent to shorelines, consist of pairs of sluice gates (north and south): at Cascades Island, SW-SG-3N and -3S shall be closed at all tailwater elevations.  At B-Branch, when tailwater exceeds 17’, SO-SG-4N and -4S shall be closed. When tailwater is between 9’ and 17', sluice gate SO-SG-4N shall be open. When tailwater is below 9', both sluice gates SO-SG-4N and -4S shall be open.

Comments from others:

Record of Final Action: