# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	15BON006 – PH2 Mid-Range Operation Dates

Date Submitted:		November 10, 2014

Project:			BON

Requester Name, Agency:	Scott Bettin, BPA

Final Action:

FPP Section:  BON 5.2. - Turbine Unit Operating Range.

Justification for Change:  A request to end the mid-point operation during low flows. Having to bring on and off a unit worth 13 kcfs when you only want to make a 2 kcfs change is too disruptive. A date even earlier than August might be needed but I was trying to postpone it as long as possible.

Proposed Change:

  - Turbine Unit Operating Range.

5.2.1. Turbine unit operations within ±1% of peak turbine efficiency (1% range) are specified in the BPA Load Shaping Guidelines (Appendix C) for implementation during the period of April 1 through October 31.  Through regional coordination with FPOM and TMT, the 1% range guidelines during this period have been modified as defined below in 5.2.1.1 to minimize PH2 gatewell turbulence for bypassed juvenile salmonids until structural and/or other solutions are implemented.   Turbine unit operating range limits are defined in Tables BON-15 (PH1) and BON-16 (PH2).

5.2.1.1. From April 1 through August 1, turbine units will operate sequentially in the following order of operating ranges to pass increasing levels of flow:

5.2.1.1.a. PH2 units within 1% mid-range;

5.2.1.1.b. Then, PH1 units up to 1% upper limit;

5.2.1.1.c. Then, PH1 units up to BOP;

5.2.1.1.d. Then, additional flow in excess of what can be passed in steps above will be passed in one of the three following ways, or as otherwise determined by Project Fisheries based on observed conditions:

d.1. April 1–April 9; June 16–  August 1: PH2 units up to 1% upper limit.

d.2. April 10–June 15 (Spring Spill) w/ Juvenile Trigger1: When juvenile spring Chinook collection counts at BON JMF are greater than adult spring Chinook total passage counts (excluding jacks) for three consecutive days, Project Fisheries will notify the control room to maintain PH2 units within 1% mid-range as a hard constraint and pass additional flow as spill.

d.3. April 10–June 15 (Spring Spill) w/ Adult Trigger: When adult spring Chinook total passage counts (excluding jacks) are greater than juvenile spring Chinook collection counts at BON JMF for two consecutive days, Project Fisheries will notify the control room to increase PH2 up to 1% upper limit in priority order from north to south: 18, 17, 16, 15, 14, 13, 12, 11.

Comments from others:

Record of Final Action:

Adult spring Chinook: reported as “Adult Chinook daily” in Corps Adult Fish Count Running Sum Report for Bonneville, available online at: http://www.nwp.usace.army.mil/Missions/Environment/Fish/Data.aspx