# Fish Passage Plan (FPP) Change Request Form

Change Form # & Title:	16BON002 – PH2 Mid-Range Operation

Date Submitted:		17-Dec-2015

Project:			BON

Requester Name, Agency:	Scott Bettin, BPA

Final Action:

FPP Section:  BON 5.2.1 - Turbine Unit Operating Range.

Justification for Change:  Change form 15BON006 was only approved for the 2015 FPP based on the assumption that PH2 units would rarely operate above the 1% mid-range in August, consistent with previous years (see FPC Memo 43-15, March 13, 2015).

The tables below summarizes hourly average PH2 turbine flow data for August 1-31, 2015.  This change form confirms the BON turbine operating range Apr 1-Oct 31 for 2016 and beyond, until PH2 unit FGE modifications are complete. Unit 15 modifications were completed in 2015 prior to testing, and the rest of the units will be outfitted with the same configuration.

| August 1-31, 2015 Summary | Unit 11 | Unit 12 | Unit 13 | Unit 14 | Unit 15 | Unit 16 | Unit 17 | Unit 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX Hourly Flow (kcfs) | 16.1 | 14.8 | 14.1 | 13.8 | 0 | 0 | 15.4 | 16.1 |
| Total # Hours Operated | 608 | 133 | 21 | 3 | 0 | 0 | 423 | 199 |
| Hrs at 15.1 - 15.5 kcfs | 12 hrs (2.0%) | 0 | 0 | 0 | 0 | 0 | 10 hrs (2.4%) | 1 hr (0.5%) |
| Hrs at 15.5 - 16.0 kcfs | 5 hrs (0.8%) | 0 | 0 | 0 | 0 | 0 | 0 | 10 hrs (5.0%) |
| Hrs at 16.0 - 16.5 kcfs | 2 hrs (0.3%) | 0 | 0 | 0 | 0 | 0 | 0 | 3 hrs (1.5%) |
| TOTAL Hrs > Mid-Range | 19 hrs (3.1%) | 0 | 0 | 0 | 0 | 0 | 10 hrs (2.4%) | 14 hrs (7.0%) |

| Aug 2015 Hrs >Mid | Unit 11 |  |  | Unit 17 |  |  | Unit 18 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Op Hrs/Day | # >Mid | % >Mid | Op Hrs/Day | # >Mid | % >Mid | Op Hrs/Day | # >Mid | % >Mid |
| 8/13/15 | 24 | 0 | 0.0% | 24 | 1 | 4.2% | 0 |  |  |
| 8/14/15 | 24 | 2 | 8.3% | 24 | 2 | 8.3% | 0 |  |  |
| 8/15/15 | 24 | 0 | 0.0% | 21 | 0 | 0.0% | 0 |  |  |
| 8/16/15 | 24 | 0 | 0.0% | 24 | 3 | 12.5% | 0 |  |  |
| 8/17/15 | 24 | 2 | 8.3% | 24 | 1 | 4.2% | 0 |  |  |
| 8/18/15 | 24 | 3 | 12.5% | 24 | 0 | 0.0% | 0 |  |  |
| 8/19/15 | 24 | 0 | 0.0% | 24 | 3 | 12.5% | 0 |  |  |
| 8/22/15 | 24 | 2 | 8.3% | 0 |  |  | 24 | 2 | 8.3% |
| 8/23/15 | 24 | 5 | 20.8% | 0 |  |  | 24 | 5 | 20.8% |
| 8/25/15 | 24 | 1 | 4.2% | 11 | 0 | 0.0% | 24 | 3 | 12.5% |
| 8/26/15 | 13 | 4 | 30.8% | 0 |  |  | 13 | 4 | 30.8% |

Proposed Change: See edits in track changes on next page.

- 5.2. 	Turbine Unit Operating Range.

5.2.1. From April 1 through October 31, turbine units are operated within ±1% of peak turbine efficiency (1% range), as specified in the BPA Load Shaping Guidelines (Appendix C).  Turbine unit operating range limits are defined in Table BON-15 (PH1) and Table BON-16 (PH2).

Through regional coordination with FPOM and TMT, the 1% range guidelines during this period have been modified as defined below in 5.2.1.1–5.2.1.2 to minimize PH2 gatewell turbulence for bypassed juvenile salmonids until structural and/or other solutions are implemented.

Unit 15 was structurally modified and tested in 2015 for the B2 FGE study. Based on the results, all other PH2 units will be modified with the same configuration. *Units that have been modified (e.g., Unit 15) may be operated within the full 1% range April 1-October 31, and the restrictions below do not apply.

5.2.1.1. April 1–July 31: turbine units will operate in the following order of operating ranges to pass increasing flow:

5.2.1.1.a. PH2 units within 1% mid-range (*units that have been modified for FGE improvements may be operated within the full 1% range);

5.2.1.1.b. Then, PH1 units up to the 1% upper limit;

5.2.1.1.c. Then, PH1 units up to the Best Operating Point (BOP);

5.2.1.1.d. Then, additional flow in excess of what can be passed in steps above will be passed in one of the three following ways, or as otherwise determined by Project Fisheries based on observed conditions:

d.1. April 1–April 9  /  June 16–July 31: PH2 units up to the 1% upper limit.

d.2. April 10–June 15 (Spring Spill) w/ Adult Trigger1: When adult spring Chinook total passage counts (excluding jacks) are greater than juvenile spring Chinook collection counts at BON JMF for two consecutive days, Project Fisheries will notify the control room to increase PH2 up to the 1% upper limit in priority order from north to south: 18, 17, 16, 15, 14, 13, 12, 11.

d.3. April 10–June 15 (Spring Spill) w/ Juvenile Trigger1: When juvenile spring Chinook collection counts at BON JMF are greater than adult spring Chinook total passage counts (excluding jacks) for three consecutive days, Project Fisheries will notify the control room to maintain PH2 units within the 1% mid-range as a hard constraint and pass additional flow as spill (*units that have been modified for FGE improvements may be operated within the full 1% range).

5.2.1.2. August 1–October 31*: PH2 units may be operated within the full 1% range and PH1 units may be operated up to BOP.

Comments from others:

From: Rerecich, Jonathan G NWP 
Sent: Tuesday, November 17, 2015 6:21 AM
To: Wright, Lisa NWD 
Subject: RE: BON Unit 15

Yes, the high carbon steel prototype 2014 plates were swapped with stainless last spring, 2015, prior to testing.  Unit 15 configuration is considered complete at this point and plans moving forward to outfit the rest of the units with the same config.  We are close to a 90% DDR which should be sent to FFDRWG for review and comment late Nov.  DDR review will be discussed with the region at a following FFDRWG where we anticipate getting regional support moving forward.  The FPP change form no 1% restriction is totally appropriate.

-----Original Message-----

From: Wright, Lisa NWD

Sent: Monday, November 16, 2015 3:30 PM

To: Rerecich, Jonathan G NWP

Subject: BON Unit 15

Hi Jon, Is BON Unit 15  considered complete for the FGE modifications? If so, I'll specify that in the FPP change form that Unit 15 is not restricted to the mid-range and can operate in the full 1%.

Record of Final Action: