OFFICIAL COORDINATION REQUEST FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 19 LMN 03 Spillway Gate 7 Outage

COORDINATION DATE- 3/20/19

PROJECT- Lower Monumental Dam

RESPONSE DATE- 4/2/19

1. Description of the problem

During routine climbing inspections of spillway gates 7 and 8 following the end of spill in 2018, inspectors discovered a bent structural member on Gate #7.  Structural Engineers from NWW have performed calculations to determine whether the spillway gate is structurally sound for continued operation with inconclusive results, necessitating the need to perform testing.  Testing to measure stresses on various structural members is scheduled to occur during the last week of March.  Results and analyses from this testing to resolve operational capability for 2019 won’t be available before early May 2019 so spring spill at Lower Monumental will need to commence without Spillway Gate 7 available for operation.

2. Type of outage required – None

3. Impact on facility operation –Alternative spill patterns have been developed in the event that Spillway Gate #7 remains unavailable throughout 2019 spill season.  They are attached to this MOC.

4. Impact on unit priority – None.

5. Impact on spill – Spring spill at Lower Monumental will commence without Spillway Gate 7.

6. Dates of impacts/repairs – The outage of Spillway Gate #7 could extend throughout the entire 2019 Spring and Summer spill periods.

7. Analysis of potential impacts to fish – Adult fish may experience different tailrace conditions than previous years when spilling in uniform pattern. Amount of spill through spill gates will remain the same.

8. Expected Impact Analysis – Previous studies have shown a small proportion of juveniles pass the dam through Spillway Gate 7. Most fish pass through the RSW or other spillway gates where survival is also very high. No anticipated effects on juvenile outmigration are expected. Attached alternate spillway patterns have been developed by a hydraulic engineer to alleviate any possible effects on tailrace hydraulics and adult upstream passage. No adult passage delay is anticipated.

9. Comments from agencies

10. Final coordination results

11. After Action update

Please email or call with questions or concerns.

Thank you,

Chuck Barnes

Supervisory Fish Biologist

Lower Monumental Dam

(509) 282-7211

Charles.a.barnes@usace.army.mil