OFFICIAL COORDINATION REQUEST FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 19 LWG 01 Unit 2 Model Validation

COORDINATION DATE- 20 March 19

PROJECT- Lower Granite Dam

RESPONSE DATE- April 2, 2019

Description of the problem- LWG Unit 2 model validation is required to verify the generator capability curve following the completion of digital governor installation.  Model validation ensures the unit operates and responds correctly and is required prior to returning the unit to service.  LWG 18 MOC 10 coordinated for OPTO 22 and digital governor installation and testing but did not identify the time needed for model validation.  Model validation is schedule to occur April 4 with the unit operating at various loads over a four hour period to generate the capability curve.  This will require operating unit 2 at the low and high ends of its range resulting in a deviation from 1% operating range.  Unit 2 will be operated outside of unit priority order during modeling.  Dates coordinated in LWG 18 MOC 10 for OPTO 22 and on digital governor installation/testing are shown in table 1.

Table 1. Tentative Unit OPTO 22 Upgrade Schedule.

| Unit | Dates |
| --- | --- |
| 1 | February 11-20, 2019 |
| 2 | March 25- April 5, 2019 |
| 3 | November 19-30? (TBD) |
| 4 | January 7-18, 2019 |
| 5 | October 1-12, 2018 |
| 6 | September 10-21 |

Type of outage required- N/A

Impact on facility operation (FPP deviations) – Model validation will require deviating from unit priority order and the 1% operating range while the unit is operated at different MW loads. Unit priority will be impacted during modeling.

Impact on unit priority- Model validation will require unit 2 to be brought on line outside of unit priority for starts, stops, and load changes as coordinated in LWG 18 MOC 10.

Impact on forebay/tailwater operation- Tailwater conditions while change as MW loading changes during the model validation.

Impact on spill- N/A

Dates of impacts/repairs- April 4, 2019.

Length of time for repairs- Model validation is expected to take up to four hours to complete with unit 2 intermittently operated outside the 1% operating range.

Analysis of potential impacts to fish

- 10-year average passage by run during the period of impact for adults and juvenile listed species, as appropriate for the proposed action and time of year; Steelhead and fall Chinook return rates are expected to be below the 10 year average during 2018 and 2019 return seasons.  Ten year daily averages adult fish passage is 186 steelhead.  Ten year average juvenile passage for April 4 is 15,824 juvenile salmonids.

Table 2. Estimated ten year daily average adult fish passage and daily percent of adults that may be impacted by species.

| LGR ADULTS |  | 10 Year Daily Average Passage |  |  | Percent of Run Impacted |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | Outage Date | Steelhead | Chinook | Coho | Steelhead | Spring Chinook | Fall Chinook | Coho |
| 2 | 25Mar-05Apr 2019 | 204 | 0 | 0 | 0.12% | 0.00% | 0.00% | 0.00% |

*Note model validation 4 hours and averages and percent of runs are calculated for the full 24 hour day.

Table 3.  Estimated ten year daily average juvenile fish passage and daily percent of juvenile fish passage by species.

| LGR JUVENILES |  | 10 Year Daily Average Passage* |  |  |  |  | Percent of Run Impacted* |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | Outage Date | CH1 | CH0 | STH | SOC | Coho | CH1 | CH0 | STH | SOC | Coho |
| 2 | 25Mar-05Apr 2019 | 6385 | 102 | 9494 | 248 | 34 | 0.18% | 0.12% | 3.52% | 4.91% | 0.04% |

*Note model validation 4 hours and averages and percent of runs are calculated for the full 24 hour day.

- Statement about the current year’s run (e.g., higher or lower than 10-year average); Steelhead and fall Chinook are expected to return at a rate below the 10 year average during the 2019 fish passage seasons due to poor ocean conditions.  As of March 16 adult steelhead passage is 10.7% of the ten year average passage at Lower Granite.

- Estimated exposure to impact by species and age class (i.e., number or percentage of run exposed to an impact by the action);  The percent of adult passage that may be impacted by model validation are listed by species in Table 2 and the percent of juvenile fish passage by species are listed on Table 3.

- Type of impact by species and age class (increased delay, exposure to predation, exposure to a route of higher injury/mortality rate, exposure to higher TDG, etc.); Operating times at each MW interval will be brief lasting enough time to bring the unit up to the MW required for modeling.  Changes in tailrace conditions related to commissioning the units may result in brief adult fish passage delays.  Impacts to juvenile passage are expected to be minimal as modeling will take place during the day and passage/collection generally increases at night.

Summary statement - expected impacts on:

Downstream migrants: Minimal impacts are expected.

Upstream migrants (including Bull Trout): Minimal impacts are expected.

Lamprey: Minimal impacts are expected.

Comments from agencies:

Final coordination results:

After Action update (After action statement stating what the effect of the action was on listed species. This statement could simply state that the MOC analysis was correct and the action went as expected, or it could explain how the actual action changed the expected effect (e.g., you didn’t need to close that AWS valve after all, so there was no impact of the action).  List any actual mortality noted as a result of the action)

Please email or call with questions or concerns.

Thank you,

Elizabeth Holdren

Supervisory Fisheries Biologist

Lower Granite Lock and Dam

Ph. 1(509)843-2263

Elizabeth.a.holdren@usace.army.mil