Change Request Number: 12TDA003 Table TDA-4 unit priority.

Date:  12/15/2011

Proposed by: Paul Keller

Location of Change:  The Dalles Dam

Proposed Change: On page TDA-19, Table TDA-4 in the FPP states from December 1 through December 15, unit priority is 1 and/or 2, 3 and/or 4,8,18. Unit 8 should not be included since it does not have an open sluicegate. Unit 3 and/or 4 can probably be deleted since west loading is intended for juvenile passage season. May need more discussion.

Table TDA-4. Turbine unit operating priorities for The Dalles Dam.. PERIOD; PRIORITY. Fish Passage Season; 1 and/or 2, 3 and/or 4, 8, 18*. (April 1 through November 30). If more units are needed operate one unit from; block 5-8, block 9-12,. each block moving west to east; block 13-16, block 17-22. If additional units are still needed operate one; block 5-8, block 9-12,. unit from each block moving west to east.; block 13-16, block 17-22. December 1 through December 15; 1 and/or 2, 18†. December 16 through February 28; 1-22 in any order. March 1 through March 31; 1 and/or 2, 3 and/or 4, 8, 18†. *During fish passage season-Unit 1 and/or 2, Unit 3 or 4, Units under open sluice gates 1,8,18† During the March and December operation for adult steelhead fallbacks and kelt passage – Unit 1 and/or 2 and. Unit 18 must be operated under at least 2 open sluice gates per unit.

Reason for Change: This is suspected a leftover requirement from juvenile passage. Reviewing Battelle’s study “Hydroacoustic Evaluation of Overwintering Summer Steelhead Fallback and Kelt Passage at The Dalles Dam 2009-2010, 3.2.1.2.Horizontal Distribution:

“Fallback of overwintering summer steelhead was highest at Sluice 1 (482 targets). Sluice 18 had the second highest number of fish passing (321) and a small number passed through the powerhouse MU 8, 16, and 18 (25, 38, and 12 fish, respectively) Figure 3.4). For individual sluice entrances, Sluice 1-3 passed the highest number of fish (469 targets), followed by Sluice 18-2 (293). Fewer than 50 fish passed through Sluice 1-2, and 18-1 (Figure 3.5)”.

No fish passed through MU3 and MU4.

Comments from others:

Record of Final Action: approved at the 20 January 2012 FPP meeting.