CENWP-OP-B								16 January 2002

MEMORANDUM FOR:	THE RECORD

SUBJECT:	Result of transport pipe inspection.

- While biking through the pipe, the following deformities were found and noted.  Distances are approximate.

40' downstream of hatch #5 the pipe has compressed to 40" high by 48" across.

50' downstream of hatch #5 the pipe has compressed to 39" high by 48" across.

60' downstream of hatch #5 the pipe measures 43" high by 46" across.

70' upstream of hatch #6 the pipe measures 39" high by 49.5" across.

50' upstream of hatch #8 the pipe measures 46" high by 41" across.

- Inside the pipe 6' upstream of hatch #9 are 3 gouges measuring ¼” deep and 3" long.  These are located below the water line.

- In addition to the above deformities, we also noted that the seams between the vertical and horizontal pipes at hatch #9 and at hatch #17 are coming apart enough to leave a noticeable gap.

- Other than the items mentioned above, the pipe appeared to be in good condition with a healthy layer of mud and slime along the bottom and sides.

- No fish were seen in the pipe, a three-spined stickleback was rescued just below the upstream switchgate.