OFFICIAL COORDINATION REQUEST FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 13BON06 CI fishway noise issue

COORDINATION DATE-  4/17/2013

PROJECT-  Bonneville

RESPONSE DATE-  ASAP

Description of the problem: The side entrance of the Cascades Island adult ladder, which is no longer used, has a metal frame associated with it that is making a tremendous amount of noise since spill started. Project biologists and the NOAA Fisheries inspector feel that it is a significant passage impediment to adults so the project would like to remedy it ASAP.

The project will first attempt to pull the frame completely out with a crane. If this is unsuccessful, personnel will be lowered into position to temporarily weld the structure to eliminate the noise until it can be permanently removed during the next winter maintenance period.

Type of outage required: If the Project cannot remove the frame, Spillbay 1 will need to be closed while personnel are lowered down to weld the frame in place.

Impact on facility operation: Spillbay 1 would be closed and the spill pattern would not match the FPP pattern.

Dates of impacts/repairs: The Project will attempt to remove the frame on 18 April 2013.  If removal does not work, they will immediately set about closing spillbay 1 and welding the frame.

Length of time for repairs: one work day (likely significantly less than this)

Expected impacts on fish passage:  Impacts to fish are potentially significant now, with the noise in the fishway from the frame rattling.

Closing Bay 1 will reduce attraction flow, but with the rest of the spillway in full operation, that impact is likely minimal.

Cascades Island will still be in normal operation.

Comments from agencies

NOAA- -----Original Message-----
From: Gary Fredricks - NOAA Federal [mailto:gary.fredricks@noaa.gov] 
Sent: Thursday, April 18, 2013 11:41 AM
To: Mackey, Tammy M NWP
Subject: Re: FPOM: MOC 13BON06 (UNCLASSIFIED)

Tammy,  Ben and I talked about this one.  I obviously agree with the outage and the need.  Thanks,  Gary

CRITFC- -----Original Message-----
From: Tom Lorz [mailto:lort@critfc.org] 
Sent: Thursday, April 18, 2013 11:41 AM
To: Mackey, Tammy M NWP
Subject: Re: FPOM: MOC 13BON06 (UNCLASSIFIED)

let us know how this worked out and if we need to deal with later if the weld option fails.  Hope you are able to remove to make all of our lives easy.

tom

NWP- PM-E-

-----Original Message-----
From: Tackley, Sean C NWP 
Sent: Thursday, April 18, 2013 8:58 AM
To: Mackey, Tammy M NWP
Subject: RE: FPOM: MOC 13BON06 (UNCLASSIFIED)

Classification: UNCLASSIFIED

Caveats: NONE

Glad to see this taken care of.  I agree that this may act as a passage deterrent and that fixing it outweighs the effects of a brief change in operations.

Thanks,

Sean

Final results- the upper sections of the frame were removed on 18 April.

A similar condition is occurred at B-Branch.  The Project removed the B-Branch frame sections the week of 13 May.

Please email or call with questions or concerns.

Thank you,

Tammy

Tammy Mackey

NWP Operations Division Fishery Section

Columbia River Coordination Biologist

503-961-5733

Tammy.m.mackey@usace.army.mil