OFFICIAL COORDINATION REQUEST FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 12TDA09 TDA ROV sill inspection.

COORDINATION DATE- Oct 25, 2012

PROJECT- The Dalles Dam

RESPONSE DATE- Nov 8, 2012 (FPOM)

Description of the problem: In preparation for this year fishway dewaterings, bulkhead sills on the north and south side of the spillway need to be ROV inspected for debris before placing dewatering bulkheads. With the volume of spill last season, some rock debris could exist on the sill. To complete this task the flow needs to be reduced at these locations to effectively operate the ROV. If we wait until the bulkheads are placed, debris will prevent seal and the delay will greatly impact this winter maintenance schedule.

Type of outage required: Auxiliary water supply - East fishway Fish Units 1 and 2 AND north fishway PUD turbine.

Impact on facility operation: Fishway attraction flow will be reduced or off.

Dates of impacts/repairs: For TDA-E south entrance – 14 November.  For TDA-N entrance – 17 December.

Length of time for repairs: 1 hour each.

Expected impacts on fish passage:

For the east fishway south entrance sill on Nov 14, it is still officially fish passage season. No fish count data exists 2008 to present. On this date in 2007, total fish passage at The Dalles was 40 Chinook, 65 steelhead and 5 Coho. The north fishway will be in operation during this work.

For the north fishway sill on Dec 17, it is not in the fish passage season, however the north fishway will be out of criteria at the same time the east fishway is dewatered for maintenance. One fish passage system is required in criteria operation during winter per Fish Passage Plan.

Comments from agencies

-----Original Message-----
From: Tom Lorz [mailto:lort@critfc.org] 
Sent: Monday, October 29, 2012 11:03 AM
To: Mackey, Tammy M NWP
Subject: Re: FPOM: Official Coordination- 12TDA09 ROV sill inspection(UNCLASSIFIED)

What time of day to you expect the 1 hour outage???  Do this later in the day, ie low adult passage time, and no issue from us.

thanks

tom lorz

-----Original Message-----
From: Cordie, Robert P NWP 
Sent: Monday, October 29, 2012 11:18 AM
To: Mackey, Tammy M NWP
Subject: RE: FPOM: Official Coordination- 12TDA09 ROV sill inspection(UNCLASSIFIED)

Classification: UNCLASSIFIED

Caveats: NONE

I should have put that in there. 0700-0800 fits the diel graph best for fall chin and B steelhead. I'll bring that to FPOM.

8 November 2012 FPOM.  12TDA09 ROV sill inspection.  Cordie reported anytime before 0800 would be a better option, based on the diel passage information.  This action would affect TDA-E east entrance and west entrance.  FPOM approves with the understanding that this work will NOT occur first thing in the morning.  Late morning/early afternoon is preferred.

Final results- This activity will go forward as coordinated, with a late morning/early afternoon start time.

Please email or call with questions or concerns.

Thank you,

Tammy Mackey

NWP Operations Division Fishery Section

Columbia River Coordination Biologist

503-961-5733

Tammy.m.mackey@usace.army.mil