RESEARCH PACKET

AND

BONNEVILLE PROJECT IMPACTS DOCUMENT

NOAA FISHERIES RESEARCH ACTIVITIES

2017

EVALUATION OF PIT TAGS AT THE CORNER COLLECTOR

NOAA Contact:	Gordon Axel, (509) 539-1270

Sponsor:		BPA

Fish Ecology Division

National Marine Fisheries Service (NOAA Fisheries)

3305 E Commerce St.

Pasco, WA  99301

(509) 539-1270

19 February 2017

EVALUATION OF CORNER COLLECTOR PIT-TAG DETECTION EFFICIENCY

BACKGROUND

The fisheries community in the Columbia River Basin, through research funded primarily by Bonneville Power Administration (BPA) and the US Army Corps of Engineers (Corps), tags around 2 million salmonids each year with FDX-B PIT tags.  Most of these fish are tagged with the standard size PIT tag, which currently is 12.34 mm in length by 2.04 mm in diameter.  BPA contracts NOAA (National Oceanic and Atmospheric Administration) Fisheries and PSMFC (Pacific States Marine Fisheries Commission) to evaluate the potential impact of adopting alternative technologies, such as new tag models, before the technology is adopted or installed.  This is done because, once integrated, these new technologies can have a significant impact on how well the installed interrogation systems detect tagged juvenile and adult salmonids.

NOAA Fisheries and PSMFC work together to evaluate the potential impact of adopting alternative technologies because the agencies have different responsibilities.  PSMFC's responsibility is related to operations and maintenance and NOAA Fisheries’ responsibility is to ensure that a technology is appropriate for fisheries applications and that the new technologies perform as advertised.  This partnership ensures that a technology is ready to move from research and development into operations before it is adopted or installed.

Over the years, BPA has periodically requested that PSMFC and NOAA Fisheries evaluate different models of FDX-B PIT tags in the 9-12 mm size to determine if they will perform acceptably in the network of interrogation systems installed throughout the Columbia River Basin (check out the PTAGIS Website at www.ptagis.org/ for a description of the sites and their antennas).

One part of evaluating the proposals will be a series of laboratory and field tests that will be conducted to determine if candidate tags could be acceptably detected by the network of PIT-tag systems.  The Bonneville Dam corner-collector PIT-tag system (BCC) has been the concept design and preliminary field testing location for PIT tag and detection system development.  The BCC system is comprised of the largest functional FDX antenna (17’ x 17’) with a specialized transceiver designed specifically for this application.  NOAA Fisheries has utilized this system to field test new PIT tag models and transceivers.

We have requested up to 10,000 juvenile tule fall Chinook Salmon from Spring Creek NFH to evaluate the PIT tags submitted by vendors for consideration by BPA for contracting.  Total number of fish required for tests will be determined in February/March based on how the number of tag models which have met lab testing requirements.

OBJECTIVES

We will use 10,000 fish tagged with PIT tags to evaluate detection efficiencies for each type of tag/vendor using the BCC detection system.  With a detection rate of 75-80%, fish will yield an accurate estimate of reading efficiencies within ±3%.

METHODS

We plan to run the fish tests in as soon as the corner collector is opened full time for the year because the hatchery will be releasing its fish around that same time.  We are planning on using tule fall Chinook salmon that we will collect from Spring Creek National Fish Hatchery or whichever hatchery USFWS grants us fish from for our evaluation.  The fish will be picked up by NOAA Fisheries and trucked to Bonneville Dam.  We will then tag them in the juvenile fish facility.

SCHEDULE

Work would be scheduled as follows:

Completion date				Task

April 24, 2017	Transport fish from Spring Creek NFH to JFMF for holding.

April 24, 2017	Install fish release hose on intake deck at BCC.

April 24-25, 2017	Tag fish at JFMF.

April 26, 2017	Begin fish releases at Second Powerhouse Corner Collector.

April 27, 2017	Complete fish releases at Second Powerhouse Corner Collector and remove research gear.

PIT-TAGGING JUVENILE HATCHERY TULE FALL CHINOOK FOR BCC TRANSCEIVER TEST

Juvenile hatchery fall Chinook salmon will be provided by Spring Creek National Fish Hatchery and held at the JFMF.  Fish will be anesthetized, weighed, measured, and PIT-tagged at the JFMF, and then allowed to recover prior to release to the river.

FACILITIES AND EQUIPMENT REQUIREMENTS

Holding and tagging of test fish will require use of existing JFMF facilities.  Temporary provisions for holding fish, e.g., containers, water supply plumbing, and drainage piping, will be set up at the JFMF.

Release of test fish at the Second Powerhouse Corner Collector will require access to the Second Powerhouse intake deck.  On the intake deck, use of the river water supply will be necessary to provide flushing water during morning fish releases and also to provide holding and release hose water.

PROJECT IMPACTS (ALL ACTIVITIES)

Project Services

No project services will be needed for this evaluation.

Security

Although most activities will occur during daylight hours, we anticipate study personnel may need to be on the Bonneville Project grounds outside of normal working hours.  Since most tasks will take place in secure project locations, contact with the public will be limited to transit between the Second Powerhouse and the juvenile facility.

Partial lists of personnel and vehicles are shown in Tables 1 and 2.  Other employees and vehicles may be added prior to start up.  First aid and CPR certifications will be updated as necessary.  Updated information will be communicated to the Bonneville POC.  No boat work is involved with the project.

Table 1.  Study personnel, affiliation, activity, and certifications.

| NAME | AGENCY | ACTIVITY | FIRST AID | CPR |
| --- | --- | --- | --- | --- |
| AXEL, GORDON | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| LAMB, JESSE | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| DOWNING, SANDY | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| SANDFORD, BEN | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| MARR, RON | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| NESBIT, MATTHEW | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |
| TULLOS, LOUIS | NOAA-F | Juv. Fish Facility, BCC | 1/12/18 | 1/12/18 |

Table 2.  Vehicles.

| LICENSE | ST | DESCRIPTION | COLOR | AGENCY | OWNER | POC/CREW |
| --- | --- | --- | --- | --- | --- | --- |
| G63-1459M | US | 12 CHEVY PU | WHITE | NOAA-F | US GOV’T | LAMB, JESSE |
| G63-2107G | US | 08 FORD PU | WHITE | NOAA-F | US GOV’T | MARR, RON |
| G63-0854M | US | 12 FORD PU | WHITE | NOAA-F | US GOV’T | TULLOS, LOUIS |
| G41-1165G | US | 08 CHEVY SUV | BROWN | NOAA-F | US GOV’T | DOWNING, SANDY |
| G63-1935H | US | 09 CHEVY SUV | TAN | NOAA-F | US GOV’T | AXEL, GORDON |

Safety

All personnel will be made aware of applicable provisions of the Safety Requirements Manual #385-1-1. The Activity Hazard Analysis and the Job Hazard Analyses are attached.

| - Setup equipment at work sites. - Conduct the work. - Removal equipment. | - Foot injury - Hand injury - Eye or face injury - Noise exposure - Head injury - Hazardous substances - Material handling - Housekeeping - Fire | - Steel-toed safety shoes (05.A.08) - Appropriate hand protection (05.A.10) - Protective equipment (05.B.01, 02, 06) - Hearing protection (05.C.01, 04, 07) - Protective headgear (05.D.01, 02, 04) - Material Safety Data Sheets to be maintained on site (06.B.01) - Use of lifting devices and handling aids (14D.A.01-06) - Safe and orderly work areas (14.C.01-04, 08) - Flammable liquids handling and storage (09.b.01-04, 08, 10, 12, 14, 16, 21) Fire extinguishers (09.3.01-04) |
| --- | --- | --- |

| 1. Inject tags | - Minor personal injury - Blood-borne pathogens | - Care with injector and awareness of surroundings. - Designated, responsible employees know location and use of clean-up kit (03.A.06). |
| --- | --- | --- |

| 1. Load fish for transport | - Personal injuries | - Foot injury. Steel-toed safety shoes (05.A.08). Hand injury. Appropriate hand protection (05.A.10). Noise exposure. Hearing protection (05.C.01, 04, 07). Head injury. Protective headgear (05.D.01, 02, 04). Lifting injury. Use of lifting devices and handling aids (14D.A.01-06). |
| --- | --- | --- |
| 2. Vehicle operation | - Accident | - Seat belt use (18.B.09). - Backing (08.C.04). - Vehicle inspection (18.A.03) - Operation (18.C). |
| 3. Release fish | - Personal injuries | - Foot injury. Steel-toed safety shoes (05.A.08). Hand injury. Appropriate hand protection (05.A.10). Noise exposure. Hearing protection (05.C.01, 04, 07). Head injury. Protective headgear (05.D.01, 02, 04). Lifting injury. Use of lifting devices and handling aids (14D.A.01-06). |