OFFICIAL COORDINATION REQUEST

FOR

NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION DATE: December 1, 2008

PROJECT:  McNary spillway gate control check

RESPONSE DATE:  December 5, 2008

Description of the problem

Personnel at McNary need to test gates on December 8 through December 10, 2008.  Each gate will need to be tested individually.  Procedural steps are listed below:

1.  Take each gate to seal.

2.  Open each gate to at least 2 to 3 feet to ensure that they work through the CAM stops.

3.  Close the gate to ensure it drives through the CAM stops.

4.  Test to ensure a setpoint direction change raise -> lower and lower-> raise works properly.

The test duration for each gate should be approximately 30 minutes.

Type of outage required

No outage is required.  However, spill will occur resulting in a reduced powerhouse flows.

Impact on facility operation

Spill will occur resulting in a reduced powerhouse flows.

Length of time for repairs

The project anticipates that the work will occur December 8 through December 10.

Expected impacts on fish passage

Powerhouse flows will be reduced and spill will occur.  Impacts on fish passage are not anticipated.  However, the region is being notified of this operation because it is occurring outside of the winter maintenance period.

Comments from agencies

Final results

Please email or call with questions or concerns.

Thank you,

Tim Dykstra

509-527-7125