PROPOSAL FOR THE 2008 YAKAMA NATION SHAD FISHERY

The Yakama Nation proposes to continue in 2008 the successful shad fishery conducted at The Dalles Dam east fish ladder exit in 1996.  In 20 days of fishing that year, tribal fishers harvested over 500,000 lbs of shad having an ex vessel value of about $65,000.  This was an unprecedented volume of commercial shad harvest in the Columbia River, made all the more remarkable by the fact that not one salmonid was killed in the process.

In 1996, the Shad Fishery Task Team (a sub-group of FPOM) jointly developed a set of recommended terms and conditions under which the shad fishery should operate that were intended to minimize potential impacts of the fishery to salmonids.  Since this is a task group of FPOM, final recommendations will be at the discretion of the full FPOM team.  In addition, safety requirements for boat operation within the BRZ were included at the request of the USACE.  The proposed “rules of conduct” for the tribal shad fishery in 2008 can be broken into categories relating to the conduct of the fishery itself (e.g., time, area, gear), safety, incidental impact guidelines, and monitoring.  These are considered in turn below.

Fishery Design

The 2008 fishery will be similar to the 1996 activity in terms of gear design, fishing times, and dates, but minor changes may be incorporated on the basis of information gathered since 1996.

Gear

1.	The fishery will first utilize an L shape design containment net to increase distance from the exit while dipping. If this fails to accumulate shad, they will then utilize a modified version of the trapnet used in 1996.  The net will be anchored adjacent to one of the two exit portals at the east fishladder.  The trapnet measures approximately 20 feet long by 10 feet in width.. Floor depth tapers from 6 feet at the net entrance near the fishladder exit to about 12 feet at the upstream end to ensure that not more than half of the exit portal is occluded.  The trapnet is emptied of trapped fish by dipnets fished from small boats moored to the sides of the net.

2.	The net will be set and removed each day beginning no earlier than 10 a.m., and fishing will end no later than 9 p.m.  The fishery may operate between those hours on Monday through Friday of each week.

3.	Salmonids incidentally captured in the trapnet will be allowed to swim out of the net over the floatline.  If adult salmon must be netted, a water to water sanctuary net will be used.

4.	Fishers are required to keep all foreign odors from entering the fishladder by wearing rubber gloves to block human scent, and by ensuring that outboard motors and other sources of petroleum-based odors are kept out of the water inside a radius of 50 feet from the fishladder exit (except during emergencies).

5.	The fishery timeframe is expected to run from late May to early July.  Shad fishing may begin when shad counts at The Dalles east fishladder exceed 3,000 per day.

Safety

Tribal fishers are required to comply with boating safety requirements for operations within the BRZ.  These include the following:

1.	Approved Type I or Type III personal flotation devices for each person on board, to be worn at all times within the BRZ.

2.	At least one fire extinguisher aboard each boat at all times.

3.	At least one anchor and 200 feet of line aboard each boat at all times.

4.	Boats within the BRZ must carry a red and white flag to identify them as being part of the shad fishery.

5.	Radios, cellular phones, or pagers (as required by the USACE) must be taken into the BRZ in case of the need for emergency contact by the dam operations controller.

Salmonid Impact Limitations

The SFTT also has developed limits for impacts to salmonids by which the shad fishery should be managed. The intent of these limits is to minimize the incidental take of listed salmon and steelhead while allowing a reasonable opportunity for a shad fishery at this critical fish passage location. The parties agree that the shad fishery should be modified if:

- The incidental catch of salmon in the shad trapnet exceeds 1 per 1,000 pounds of shad in the net.

- The fallback rate for any salmonid species at the counting station increases by more than 10% of the baseline (pre-fishing) fallback rate for that species in twonon-consecutive hourly counting periods during the daily fishing period.

3. 	If fallback criteria is exceeded on 2 consecutive days, the fishery will be terminated for the remainder of the 2008 fishing season.

Fishery Monitoring

Monitoring plans for 2008 include the following:

- Fallback rates will be recorded daily by the fish counter at the east fishladder counting station.  This monitoring will begin on May 1, to obtain baseline conditions of fallback for chinook and steelhead prior to fishing and will continue during and after the fishery.

- A tribal monitor will be present on the forebay deck during periods of active fishing to record the incidence of salmonids caught in the trapnet during the fishery.  The tribal monitor will assure all regulations are clearly understood by the fishers. Incidental catch will be recorded by species, date, and time of occurrence.

- The tribal monitor will record criteria infractions and take immediate action to stop fishing activity when limits are exceeded.