ZINC GALVANIZATION vs  FISH PASSAGE

Literature Information

- Rainbow trout avoidance limit 0.0001mg/l Fish do eventually acclimate and lose avoidance behavior. (Acta Zoologica Lituanica Hydrobologia ’99 Vol 9).

- Leaching rate 5mg/cm2/day. (WES tech note ZMR-2-15, Crump) Zebra mussel anti-fouling.

- Galvanized grating likely results in concentration above rainbow trout avoidance threshold (Clugston)

- 1mg/l can impact aquatic biota (Krenkel ’79)

- Zn runoff possible cause of Coho mortalities in Seattle

- The Dalles new grating has approx 2.7mm galvanize coating (per contractor spec)

- 3mm coating provides approx 20yr corrosion protection (www.gtiengr.com)

- Zinc concentration in sea water = 0.6ppb (www.lenntech.com)

- Zinc concentration in rivers = 5-10ppb (www.lenntech.com)

- Zinc solubility depends on pH. More soluble with increased acidity (www.lenntech.com)

- Zebra Mussel fouling: galvanized steel is not susceptible to fouling. This material should be considered for retrofit, maintenance, and construction. http://el.erdc.usace.army.mil/zebra/zmis/

Questions that need to be answered

- What do salmon and steelhead avoid?

- What is Columbia river pH?

- What is the concentration in a flowing fish ladder with new grating?

- Does aging the grating on the deck reduce leaching rate when installed?

- What is background concentration in the river?

- What is the acclimation rate?

- What are alternatives to galvanized grating?

- Do alternatives leach detrimental compounds? (fiberglass resins, aluminum)

- What is the cost difference for alternatives?

- Is there a simple test (lab or field) we can conduct to determine if we have a galvanization avoidance problem?

- Two sections of galvanized grating were installed last winter. Can we determine avoidance from UofI radiotelemetry?

- If galvanization prevents zebra mussel colonization, should we take this into account?

- Can we conduct a water quality test immediately downstream of installed galvanized grating to confirm concentration levels?