ANNUAL FISHWAY

STATUS

REPORT

For

BONNEVILLE DAM

2002

By

Jennifer D. Sturgill

Tammy M. Mackey

Timothy J. Darland

U. S. Army Corps of Engineers

CENWP-OP-B

Bonneville Lock & Dam

Cascade Locks, OR 97014

(541) 374-8375

28 January 2002

TABLE OF CONTENTS

Introduction

2

FISHWAY oPERATION AND aCTIVITIES	3

FISH PASSAGE PLAN (FPP) VIOLATIONS	4

Auxiliary Water System (AWS) Closures	5

Fish UNIT Outages	5

Fish FACILITY Outages and Maintenance	5

TURBINE OUTAGES AND DEWATERINGS	6

fISH REMOVAL	6-7

VBS/STS INSPECTIONS

Table 1.  Submersible Traveling Screen (STS)and Vertical Barrier Screen (VBS) Inspections	8

ZEBRA Mussels	8

GLOSSARY	9

REFERENCES	10

# INTRODUCTION

This document summarizes activities impacting fish at Bonneville Lock & Dam from 1 December 2001 through 30 November 2002.

Primarily a summary of weekly reports, this document is required by the Corps of Engineers’ North Pacific Division, as described in the Fish Passage Plan (FPP) .  The FPP contains the following surveillance and reporting requirements.  “Project biologists shall inspect fish passage facilities at the frequencies listed in the juvenile and adult fish facilities operating criteria sections.  The (weekly) reports shall include: any out of criteria situations observed and subsequent corrective actions taken; equipment malfunctions, breakdowns or damage, along with a summary of resulting repair activities; adult fish control calibrations; STS and VBS inspection; any unusual activities which occurred at the project which may affect fish passage.”

Figure 1 Bonneville Lock & Dam.

The project and fish passage facilities are described in the Fish Passage Plan1.  In summary, the project comprises two powerhouses, a spillway and two navigation locks.  The older of the two navigation locks has not operated since early 1993.  There are four adult fish ladders for upstream migration, one at PH1, one at the south end of the spillway, one at the north end of the spillway and one at PH2.  There are two Juvenile Bypass Systems for downstream migration, one at each powerhouse.

Acronyms and jargon are used freely in this report.  The reader can find definitions in the glossary.

# FISHWAY OPERATION AND ACTIVITIES

Fish Passage Plan (FPP) Violations

The results of 263 daily fishway inspections (8 of which were conducted by NMFS or FPC) are shown in the following table.  The FPP requires seven inspections a week during fish passage season.  Project Fisheries conducts two inspections each day and the control room conducts the third.  Shown are the FPP violations, numbers of days the violation occurred, and the percentage of days the item was IN criteria.

## Auxiliary Water System (AWS) Closures

The AWS’ were closed for trashrack cleaning on several occasions during the 2001-2002 reporting year. Closures ranged from four minutes to over five hours with an average closure lasting 38 minutes.  AWS trashracks were usually cleaned once each month with more frequent cleanings during late summer and early fall.  This year FV3-9 had 17 closures with an average of 23 minutes per cleaning.  FV3-7 had nine closures with average closure time of 41 minutes.   FV1-1 was closed 19 times with an average closure lasting 5 minutes.  The valve was also shut down during the evening to float debris in late November.  Mid season dives and ladder outages are not included in cleaning closure times.

FV4-3, FV4-4, FV5-3 and FV5-4 were occasionally closed for up to 20 hours to facilitate Washington Shore log boom repair, research equipment installation, and mid-season fishway inspection dives.

## Fish Unit Outages

03 Dec 01 – 13 Mar 02	F1 OOS for annual maintenance and PH2 ladder dewatering.

20 Mar 02 – 20 Mar 02	F1 OOS for PH2 Collection Channel velocity meter installation.

16 Apr 02 – 16 Apr 02	F1 OOS for trashrack raking.

22 May 02 – 23 May 02	F1 OOS for exciter maintenance and stator measurements.

06 Jun 02 – 06 Jun –02	F1 OOS for cooling water strainer maintenance.

23 Jun 02 – 23 Jun –02	F1 OOS for decontamination of turbine bearing water.

09 Jul 02 – 09 Jul 02	F1 OOS for mid-season fishway inspection and slip ring work.

14 Aug 02 – 15 Aug 02	F1 OOS for contractor measurements and testing.

20 Aug 02 – 20 Aug 02	F1 OOS for T11/12 switching.

18 Sep 02 – 18 Sep 02	F1 OOS to install contractor instrumentation.

01 Nov 02 – present	F1 OOS for overhaul and stator rewind

11 Feb 02 – 28 Feb 02	F2 OOS for annual maintenance and PH2 ladder dewatering.

20 Mar 02 – 20 Mar 02	F2 OOS for PH2 Collection Channel velocity meter installation.

16 Apr 02 – 16 Apr 02	F2 OOS for trashrack raking.

09 Jul 02 – 09 Jul 02	F2 OOS for mid-season fishway inspection and slip ring work.

20 Aug 02 – 20 Aug 02	F2 OOS for T11/12 switching.

22 Aug 02 – 22 Aug 02	F2 OOS for T11/12 switching.

01 Oct 02 – 01 Oct 02	F2 OOS for emergency repair of a failed farval line.

Fish units 1 and 2 were placed on standby at other times during the year to “float” trash away from the trash racks.  When drawdown measured one foot or greater, fish units were shut down. An adjacent unit was then operated to pull trash away from the fish unit trashracks.  This procedure helped prevent debris and silt from accumulating in front of the fish units.  Most of the unit outages associated with the floating of trash and debris occurred between midnight and 0300 to minimize impact on adult fish passage.

# Fish Facility Outages & Maintenance

29 Nov 01-- 07 Dec 01	AFC&MF (exit) OOS for PIT detector emergency repair.

07 Feb 02 -- 28 Feb 02	AFC&MF OOS for winter maintenance.

10 Dec 01 -- 02 Feb 02	Bradford Island ladder OOS for winter maintenance.

05 Feb 02 -- 08 Mar 02	Cascades Island ladder OOS for winter maintenance.

04 Oct 01 -- 27 Feb 02	DSM1 OOS for water conservation and winter maintenance.

05 Mar 02 -- 06 Mar 02	DSM1 OOS for emergency maintenance.

15 Apr 02 -- 16 Apr 02	DSM1 OOS for emergency maintenance.

23 Apr 02 -- 23 Apr 02	DSM1 OOS for emergency maintenance.

25 Jun 02 -- 25 Jun 02	DSM1 OOS for emergency maintenance.

04 Jul 02 -- 08 Jul 02	DSM1 OOS for emergency maintenance.

18 Jul 02 -- 18 Jul 02	DSM1 OOS for emergency maintenance.

29 Jul 02 -- present	DSM1 turned south for summer removal of STS.

15 Jan 02 -- 07 Mar 02	DSM2 OOS for winter maintenance.

03 Dec 01 -- 07 Dec 01	Washington Shore ladder OOS for PIT detector emergency repairs.

11 Feb 02 -- 28 Feb 02	Washington shore ladder OOS for winter maintenance.

Turbine Outages and Dewaterings (outages of at least 24 hours)

### Date Out	Date Back      Unit	Reason for Outage

2001		22-Jan-02	3	Turbine rehab

18-Jun-01	14-Nov-02	5	Turbine rehab

19-Nov-01	20-Dec-01	8	Five-year overhaul

03-Dec-01	19-Dec-01	0	Tripped offline

02-Jan-02	08-Apr-02	17	VBS slot modifications

20-Jan-02	21-Jan-02	4	Bank 3 /4 work

28-Jan-02	28-Feb-02	3	Governor rehab

26-Feb-02	21-Mar-02	10	Digital governor installation and five year overhaul

27-Feb-02	04-Mar-02	11	Bearing oil leak

13-Mar-02	14-Mar-02	6	Loop testing with Unit 5

18-Mar-02	18-Apr-02	7	Digital governor/ exciter installation

01-Apr-02	05-Apr-02	3	Governor rehab

08-Apr-02	26-Apr-02	1	Digital governor installation

08-Apr-02	10-Apr-02	15	STS work

10-Apr-02	11-Apr-02	11	STS malfunction

22-Apr-02	16-May-02	8	Digital governor/ exciter installation

23-Apr-02	25-Apr-02	6/7	Relay work

28-Apr-02	29-Apr-02	14	Breaker trouble

06-May-02	06-Jun-02	2	Digital governor/ exciter installation

06-May-02	07-May-02	17	Gantry crane failure

08-May-02	10-May-02	1	Unit 2 governor work

14-May-02	18-Jun-02	15	VBS failure

19-May-02	21-Jun-02	9	Digital governor installation and five year overhaul

21-May-02	23-May-02	1	Pre turbine rehab testing

24-May-02	26-May-02	11	STS failure

10-Jun-02			1	Turbine rehab

17-Jun-02	02-Jul-02	6	Governor modifications

25-Jun-02	27-Jun-02	0	Slip ring cutting

03-Jul-02	22-Jul-02	12	Camera caught on STS frame/exciter failure

08-Jul-02	24-Jul-02	4	Digital governor work

15-Jul-02	17-Jul-02	2	Digital governor work

24-Jul-02	07-Aug-02	7	Vibration inspection

26-Jul-02	30-Jul-02	13	Governor power failure

29-Jul-02	31-Jul-02	6	Bank 5/6 work

29-Jul-02	01-Aug-02	8	Bank 7/8 work/oil cleanup

15-Aug-02	16-Aug-02	0	Bad relay

19-Aug-02	20-Aug-02	10	transducer removal

19-Aug-02	20-Aug-02	11	T11 maintenance

19-Aug-02	20-Aug-02	12-14	T11 maintenance

20-Aug-02	21-Aug-02	11	Exciter trouble

20-Aug-02	22-Aug-02	15-18	T12 maintenance

03-Sep-02	03-Dec-02	9	overhaul

04-Sep-02	12-Sep-02	15	servo motor oil leak

18-Sep-02	19-Sep-02	17	servo motor oil leak

23-Sep-02	03-Dec-02	8	PH1 crane rail work

23-Sep-02	30-Nov-02	10	PH1 crane rail work

30-Sep-02	03-Dec-02	7	PH1 crane rail work

07-Oct-02			16	four year overhaul

08-Nov-02	09-Nov-02	6	Bank 5/6 work

# Fish Removal 2001/2002

Fish passage facilities and turbine units are taken out of service and dewatered to allow for inspection, preventative maintenance, repairs, and modifications.  As facilities and turbine units are dewatered, project biologists, outside agency personnel, and other project personnel follow procedures outlined in the Fish Passage Plan and detailed in the Fish Recovery Plan to minimize impacts on fish.  Adult salmonids are typically released into the forebay above the new navlock.  Sturgeon and juvenile salmonids are usually released below the dam at Hamilton Island.  The following is a summary of the number of fish that were removed during facility and turbine unit dewaterings.  All fish were recovered in good condition unless otherwise noted.

Washington Shore (UMT-TW) and AFF exit ladder (12-03-01): one adult steelhead.

Bradford Island Fishway Junction pool (12-10-01): 16 unclipped adult steelhead, 26 clipped adult steelhead, one clipped adult chinook, one clipped jack chinook, one lamprey, and one unclipped adult Coho, approx. 12 juvenile salmonids and many resident fish.

B-Branch diffuser pools and entrance pool (12-10/11-01): many resident fish and one five foot sturgeon.

A-Branch diffuser pools (12-10-01): nine clipped adult steelhead.

A-Branch entrance pool (01-03-02): two steelhead.

Bradford Island Junction pool (1-03-02): two peamouth, two rainbow trout.

Unit 17 draft tube (1-04-02): one dead juvenile shad and one dead juvenile lamprey.

Unit 3 tail logs (1-15-02): 14 peamouth, whitefish and suckers.

DSM2 and JMF (1-15/16-02): one stickleback.

Cascade Island (2-04-02): four adult steelhead, ten juvenile salmonids, 55 resident chubs, peamouth, and whitefish, eight adult lamprey.

UMT (02-04-02): 41 adult steelhead, couple juvenile salmonids, handful of residents.

Cascade Island diffuser pools (02-05-02): seven adult steelhead, five juvenile salmonids, 20 residents, one adult lamprey.

Washington Shore Ladder (02-11-02): 40 adult steelhead, eight steelhead smolts, 14 adult lamprey, six sculpin.

AFF (02-11-02): 68 live adult steelhead, ten dead adult steelhead (eight clipped, two unclipped), 12 steelhead smolts, six residents, one sculpin, and ten sticklebacks.

UMT (02-11-02): four adult steelhead, three residents, two steelhead smolts.

Washington Shore Ladder (02-21-02): two adult lamprey.

Cascade Island Ladder (2-26-02): 24 peamouth from behind contractor sandbags.

Unit 17 Tail logs (04-08-02): one crayfish, ten sculpin and one catfish.

DSM1 (04-15-02):  two adult spring chinook, four steelhead smolts, two chinook smolts.

DSM1 (4-23-02): one adult spring chinook, one adult steelhead, 14 juvenile steelhead, ten juvenile chinook.

Unit 9 Draft Tube (5-21-02): one juvenile salmonid and one dead sturgeon.

DSM1 (6-7-02): two sturgeon, 20 juvenile steelhead, five live juvenile chinook and 5 dead juvenile chinook.

Unit 1 Draft Tube (6-11-02): three sturgeon and 45-50 adult lamprey.

Unit 9 Tail logs (6-19-02): four sturgeon, two crawfish, two sticklebacks, two adult lamprey, and 12-15 sculpin.

DSM 1 (7-8-02): three sub-yearling Chinook, two sculpin.  Dead fish on the inclined screen.

Unit 9 draft tube (9-3-02): two sturgeon and one lamprey.

Unit 9 tail logs (10-01-02): 19 sculpin.

Unit 16 Draft Tube (10-08-02):  12 sturgeon, three channel catfish, one sculpin, one sucker.

Fish Unit 1 Draft Tube (11-01-02): one sculpin, five dead juvenile shad.

Unit 5 Tail logs (11-05-02): 44 crayfish, 69 sculpin, one smallmouth bass, one sturgeon, one stickleback, one bluegill.

Adult Fish Facility entrance ladder (11-25-02): six unknown steelhead, one unclipped steelhead, three clipped steelhead, 15 coho, two shad, two suckers, three whitefish.

Adult Fish Facility exit ladder (11-25-02): two steelhead, one sucker, four whitefish.

VBS/STS Inspections

The Fish Passage Plan requires STS inspections be conducted monthly, and VBS inspections once every two months.  Because each STS has a timer that automatically shows elapsed time of operation, STS inspection frequency can be measured by run time.  The equivalent of one month of continuous operating time is 720 hours.  Bonneville inspects both STS and VBS with an underwater video camera.  Video allows STSs and VBSs to be inspected simultaneously, and eliminates the need to dip gatewells and handle fish.

### Table 1.  Submersible Traveling Screen (STS) and Vertical Barrier Screen (VBS) Inspections 2001-2002

## Zebra Mussels

Through weekly inspections of the monitoring station at the First Powerhouse and of all dewatered fishways, we have found no indication of zebra mussel colonization.  It is understood that their arrival is inevitable.  We continue monitoring with hopes that control programs can be initiated at the first indication of the zebra mussel’s arrival in the Pacific Northwest.

# GLOSSARY

AFC&MF	Adult Fish Collection and Monitoring Facility.  Lab associated with the Washington Shore ladder.  Adult fish are trapped for research purposes.

AFF----------------Adult Fish Facility.  Also known as the AFC&MF.

BiOp	Biological Opinion.

BON	Bonneville Lock & Dam.

Downwell	Opening through which water and fish drop after excess water is drawn away through a dewatering screen.  Start of the transport conduit for fish from DSM1 to the tailrace.

DSM 1	Downstream Migrant transportation channel (PH1).  Transport channel for juvenile fish from gatewell orifices to the downwell.

DSM 2------------Downstream Migrant transportation channel (PH2).  Transport channel for juvenile fish from gatewell orifices to the juvenile transport pipe.

CRITFC	Columbia River Intertribal Fish Commission.

FFU	Fisheries Field Unit.  Unit in the Corps of Engineers’ Portland District Construction, Operations Division that conducts fish related evaluations and monitoring.

FGE	Fish Guidance Efficiency.  Proportion of fish entering turbine intakes that are guided into the juvenile bypass system.

Forebay	That area of a reservoir immediately upstream of a dam.

FPC	Fish Passage Center.

FPE	Fish Passage Efficiency.  Proportion of juvenile fish that pass the dam by non-turbine routes.

FPP	Fish Passage Plan.

JBS	Juvenile Fish Bypass System.  Usually refers to PH2.

JMF	Juvenile Fish Monitoring Facility.  Lab associated with the PH2 JBS.

Kcfs	Thousand cubic feet per second.

NDE	North Downstream Entrance.  Refers to one of the four large overflow weir adult fishway entrances at PH2.

NUE	North Upstream Entrance.  See NDE.

NMFS	National Marine Fisheries Service.

ODFW	Oregon Department of Fish & Wildlife

OOS	Out Of Service.

PCC	Powerhouse Collection Channel.  Part of the adult fishway spanning the length of the downstream side of each powerhouse.

PH1	Bonneville First Powerhouse.

PH2	Bonneville Second Powerhouse.

PIT-----------------Passive Integrated Transponder.  A tag inserted into juvenile and adult fish.  Detectors are installed at all fish passage systems.

Project	Bonneville Lock & Dam.

SDE	South Downstream Entrance.  See NDE.

SUE	South Upstream Entrance.  See NDE.

STS	Submersible Traveling Screen.

Tailwater	The portion of a river immediately downstream of a dam or powerhouse.

TIE	Turbine Intake Extension.

Tule	Early spawning fall chinook.

UMT	Upstream Migrant Transportation channel.  This channel connects Cascades Island ladder to Washington Shore ladder through PH2.

VBS	Vertical Barrier Screen.

WDFW	Washington Department of Fish & Wildlife.

# REFERENCES

?  _____. 2002.  Fish Passage Plan for Corps of Engineers Projects.  U. S. Army Corps of Engineers, North Pacific Division, Portland, Oregon.