### CENWP-OP-B                                                                                                    11 January 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 1 of 2000, which covers the period 2 through 8 January.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 188.2 to 213.1 kcfs.  Daily average tailwater elevation for the project ranged from 16.5’ to 18.0’ msl.  River temperature ranged from 43 to 46° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and B-branch ladder, respectively, remain on sill.

- Unit 10 remained out of service for stator rewind.  Unit 4 was out of service from 1030-1212 on 3 January to repair the units’ cooler.  Units 1, 2, and 3 were out of service from approximately 0700-1730 on 3, 4, 5, 6, and 7 January for divers to install surface collector equipment.  Unit 8 was out of service for testing from 1442 on 4 January through 1206 on 5 January.

- The first powerhouse ice and trash sluiceway remained open for debris removal and the second powerhouse sluiceway remained closed this reporting week.

- Second powerhouse fish units remained out of service for winter maintenance.

- ACTIVITIES.

2.1. STS Inspections.  End-of-season STS inspection data from December is presented below.  All screens were in good condition unless otherwise noted below.  See Table 1.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 8 | 12/15 | 6083 |  |
| 12 | 12/16 | 1164 | Replaced C-slot screen. |
| 13 | 12/16 | 1087 |  |
| 14 | 12/16 | 1136 |  |
| 15 | 12/16 | 735 |  |
| 16 | 12/17 | 123 |  |
| 17 | 12/17 | 718 |  |
| 18 | 12/17 | 912 | Replaced all screens. |
| 3 | 12/17 | 1259 |  |
| 1 | 12/18 | 1324 |  |
| 2 | 12/18 | 1317 |  |
| 7 | 12/18 | 1376 |  |
| 9 | 12/20 | 1355 |  |

Table 1.  STS Inspection Data.

2.2. AWS Closures.  PH1 AWS water was closed for divers to inspect the PH1 collection channel from 0815-1323 on 4 January.  FV1-1 was closed for cleaning from 1543-1559 on 6 January.

2.3. Dewaterings and Fish Salvage.  There was no fish salvage this reporting week.

- RESEARCH.

3.1. Normandeau MGR Testing.  Normandeau and Mid-Columbia Consultants resumed biological MGR testing at units 5 and 6 on 5 January.

3.2. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island.

4.  FISHWAYS.  Project biologists inspected on 3, 4, and 6 January this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | A-branch diffuser FG3-4. | Diffuser should be open at 14.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-5. | Diffuser should be open at 17.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week. | PH1 collection channel electronic velocity meter. | Meter registered <1.5 fps and should register 1.5-4.0 fps. | Unknown. | No action taken. |
| 1/3 | A and B-branch fish ladders. | Water level registered 1.4’ over the weirs and should register 1.0’+/-.1’. | Power was lost to FV3-7, causing valve to open farther than required. | Necessary adjustments were made. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 6 January.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- CONSTRUCTION UPDATE.

  - Construction continues on the new Juvenile Monitoring Facility.

6.  OTHER NOTABLE ITEMS.

6.1. Divers inspected the PH1 collection channel, Cascades Island ladder, and PH2 collection channel including north and south monoliths on 4 and 5 January this reporting week.  All facilities including diffuser gratings and gates were in good condition.

6.2. The Washington shore ladder is scheduled to return to service on 10 January.

6.3. The Cascades Island ladder is scheduled to return to service on 10 January.

6.4. The Bradford Island ladder is scheduled to be dewatered on 11 January.

6.5. The PH1 DSM is scheduled to be dewatered on 18 January.

DEBORAH L. CHENOWETH

# Operations Project Manager