### CENWP-OP-B                                                                                                    18 January 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 2 of 2000, which covers the period 9 through 15 January.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 201.7 to 227.4 kcfs.  Daily average tailwater elevation for the project ranged from 18.2’ to 19.2’ msl.  River temperature ranged from 42 to 43° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and B-branch ladder, respectively, remain on sill.

- Unit 10 remained out of service for stator rewind.  Units 1, 2, 3, and 4 were out of service from approximately 0700-1500 on 11 through 14 January for divers to install surface collector equipment.

- The first powerhouse ice and trash sluiceway remained open for debris removal.  The second powerhouse sluiceway was opened on 14 January to supply additional AWS water for the PH2 fishway due to fish unit 2 being out of service.

- Second powerhouse fish unit 1 returned to service on 10 January, fish unit 2 remains out of service for maintenance.  The unit is scheduled to return to service on 18 January.

- ACTIVITIES.

2.1.  Fish Ladder and DSM Service.  The Cascades Island and Washington shore fish ladders returned to service on 10 January.  The Bradford Island ladder including A and B-branches were taken out of service for winter maintenance on 11 January.  The PH1 DSM remained in service and the PH2 DSM and flume remained out of service.

2.2. AWS Closures.  There were no AWS closures this reporting week with the exception of Bradford Island AWS water in preparation for ladder outages.

2.3. Dewaterings and Fish Salvage.  The Bradford Island ladder including A and B-branches were dewatered this reporting week.  Project biologists recovered 13 adult steelhead, approximately 20 resident river fish, 6 juvenile salmonids and one adult lamprey from the main ladder.  Project biologists also recovered 28 adult summer hatchery steelhead, 19 adult winter hatchery steelhead, 14 adult winter wild steelhead, 9 adult summer wild steelhead, approximately 30 juvenile salmonids, approximately 20 resident river fish, and one adult lamprey from the junction pool.  Biologists also recovered an additional 16 adult steelhead and approximately 20 resident river fish from the diffuser pools.

- RESEARCH.

3.1. Normandeau MGR Testing.  Normandeau and Mid-Columbia Consultants continued biological MGR testing at units 5 and 6 this reporting week.

3.2. WDFW Fish Counting.  WDFW discontinued 24-h/d video counting at Bradford Island due to the ladder dewatering and resumed video counting at Washington shore on 10 January.

4.  FISHWAYS.  Project biologists inspected on 10 and 13 January this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week |  | Project biologists did not complete 3 fishway inspections as required per the FPP. | Additional activities. | Operators continue to inspect fishways daily. |
| All Week | A-branch diffuser FG3-4. | Diffuser should be open at 14.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-5. | Diffuser should be open at 17.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| 1/10 | PH1 collection channel electronic velocity meter. | Meter registered <1.5 fps and should register 1.5-4.0 fps. | Unknown. | No action taken. |

4.2. Sensor Calibration.  Water level sensors were not checked for calibration this reporting week.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- CONSTRUCTION UPDATE.

  - Construction continues on the new Juvenile Monitoring Facility.

6.  OTHER NOTABLE ITEMS.

6.1. The PH1 DSM is scheduled to be dewatered on 18 January.

DEBORAH L. CHENOWETH

# Operations Project Manager