### CENWP-OP-B                                                                                                  15 February 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 6 of 2000, which covers the period 6 through 12 February.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 179.2 to 234.1 kcfs.  Daily average tailwater elevation for the project ranged from 16.6’ to 18.8’ msl.  River temperature was 40° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and B-branch ladder, respectively, remain on sill.

- Unit 10 remained out of service for stator rewind.  Units 5 and 6 were out of service from approximately 0700-1700 on 7 through 11 February for divers to install surface collector equipment.  Unit 4 was also out of service from approximately 0700-1700 on 10 and 11 February for divers to install surface collector equipment.  Second powerhouse units 11-17 were out of service for approximately 1-2 hours each on 8 February to rake trash.

- The first powerhouse ice and trash sluiceway remained open according to surface collector dive locations.  The second powerhouse sluiceway remained closed with one exception.  The sluiceway was opened from 0810-1200 on 9 February to remove debris.

- Second powerhouse fish units remained in service.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1.  Fish Ladder and DSM Service.  The Bradford Island A-branch ladder and PH2 DSM and flume returned to service on 7 and 11 February, respectively.  The Bradford Island B-branch ladder and PH1 DSM remain out of service for winter maintenance.

2.2. AWS Closures.  There were no AWS closures this reporting week.

2.3. Dewaterings and Fish Salvage.  There were no dewaterings or fish salvage this reporting week.

- RESEARCH.

3.1. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Washington shore and resumed video counting at Bradford Island on 8 February.

4.  FISHWAYS.  Project biologists inspected on 8, 10, and 11 February this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP Deviation | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | PH2 collection channel velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | No action taken. |
| 2/8 | PH1 entrance gate 2. | Gate registered 7.3’ of submergence and should register >8’ per the FPP. | Automatic control failure. | Gate was adjusted accordingly. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 9 February this reporting week.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- OTHER NOTABLE ITEMS

  - The Bradford Island B-branch fish ladder is scheduled to be watered-up on 24 February.

  - The PH1 DSM is scheduled to be watered-up on 24 February.

DEBORAH L. CHENOWETH

# Operations Project Manager