### CENWP-OP-B                                                                                                  23 February 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 7 of 2000, which covers the period 13 through 19 February.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 194.9 to 224.2 kcfs.  Daily average tailwater elevation for the project ranged from 17.3’ to 18.9’ msl.  River temperature ranged from 41 to 42° F.  Spill bay 1, which supplies attraction flow for the Cascades Island ladder was opened 4” on 15 February.  Spill bay 18, which supplies attraction flow for the Bradford Island ladder, remained on sill.

- Unit 10 remained out of service for stator rewind.  Unit 18 was taken out of service at 0918 on 14 February for stator rewind.  Units 4, 5, and 6 were out of service from approximately 0700-1700 on 14 through 18 February for divers to install surface collector equipment.  Units 3, 2, and 1 were out of service from 0706-0912, 0912-1124, and 1124-1436, respectively, for researchers to install trashrack transducers.

- The first powerhouse ice and trash sluiceway remained open according to surface collector dive locations.  The second powerhouse sluiceway remained closed with one exception.  The sluiceway was opened from 1245-1551 on 15 February for debris removal.

- Second powerhouse fish units remained in service.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1.  Fish Ladder and DSM Service.  The Bradford Island B-branch ladder and PH1 DSM remain out of service for winter maintenance.

2.2. AWS Closures.  There were no AWS closures this reporting week.

2.3. Dewaterings and Fish Salvage.  Unit 18 was dewatered on 15 February for stator rewedge.  Project biologists recovered one sturgeon, one juvenile wild fall chinook, and one juvenile wild steelhead from the draft tube.  All fish were released in good condition.

- RESEARCH.

3.1. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island and Washington shore.

4.  FISHWAYS.  Project biologists inspected on 14, 15, and 17 February this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP Deviation | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | PH2 collection channel velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | No action taken. |
| 2/14, 2/15 | PH1 collection channel velocity meter. | Meter registered <1.5 fps and should register 1.5-4.0 fps. | Unknown. | No action taken. |
| 2/15 | A-branch ladder staff gauge. | Staff gauge registered .7’ over the weir and should register 1.0’+/-. 1’. | FV3-9 is closed to preclude water from overtopping weir blocks down B-branch. Consequently, water levels fluctuate down A-branch depending on forebay elevation. | No action taken. FV3-9 will be opened and adjusted accordingly when B-branch returns to service on 24 February. |
| 2/15 | PH1 gatewells 1C, 2C, 4A, 4C, 8A, 8B, 8C, and 9C. | Gatewell debris coverage was >50% and should be <50% coverage. | Increased river debris. | Gatewells are being cleaned the subsequent reporting week during STS installation. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 16 February this reporting week.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- OTHER NOTABLE ITEMS

  - The Bradford Island B-branch ladder is scheduled to return to service on 24 February.

  - The PH1 DSM is also scheduled to return to service on 24 February.

DEBORAH L. CHENOWETH

# Operations Project Manager