### CENWP-OP-B                                                                                                         13 June 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 23 of 2000, which covers the period 4 June through 10 June.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 199.4 to 258.3 kcfs.  Daily average spill ranged from 79.5 to 102.3 kcfs.  Daily average tailwater elevation for the project ranged from 16.7’ to 19.8’ msl.  River temperature ranged from 61 to 62° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

- Unit 12 remained out of service for stator rewedge and 4-year overhaul.  Units 3 and 11 were out of service from 0700-1036 and 1048-1436, respectively, on 5 June for STS inspections.  Unit 10 was out of service from 0518-1630 on 6 June for grounding ring cutting.  Unit 8 was out of service from 0730-1118 on 7 June for transducer repair and from 1524-1700 8 June because of an oil spill.  Fish Unit 2 was out of service 0236-0706 on 10 June due to an excitor that wouldn’t build voltage.

- The first powerhouse ice and trash sluiceway remained open this reporting week.  PH1 chain gates are being operated according to PSC testing requirements.  The second powerhouse sluiceway remained closed this reporting week.

- Second powerhouse fish units remained in service this reporting week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1. AWS Closures.  There were no AWS closures this reporting week.

2.2. Dewaterings and Fish Salvage.  There was no dewaterings or fish salvage this reporting week.

2.3. STS Inspections.  Two units’ STS’s were inspected this reporting week.  All screens were in good condition unless otherwise noted below.  See Table 1.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 3 | 6/5 | 614 |  |
| 11 | 6/5 | 730 | Repaired small rip in screen. |

Table 1.  STS Inspection Data.

- RESEARCH.

3.1. AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts following shad passage season.

3.2. AFC&MF Adult Sampling and Tagging.  The University of Idaho continued radio tagging and tracking adult chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult chinook and steelhead approximately 2 days per week for their age composition research.

3.3. Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington shore ladders this year.

3.4. NMFS FGE and OPE Testing.  NMFS discontinued FGE and OPE testing in unit 8B and 9B slots this reporting week.  They will resume testing on 12 June.

3.5. NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

3.6. PH2 Bypass System Evaluation.  NMFS continued periodically releasing PIT-tagged juvenile chinook and steelhead through the new bypass system this reporting week to determine if fish condition is adversely affected by system passage.  They also released juvenile sockeye in the PH2 forebay for recovery at the JMF and monitoring.  USGS continued radio-tagging juveniles this reporting week for release in the DSM and tracking from the outfall release to Beacon Rock.

3.7. PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC continued 24-h/d PIT tag interrogation at DSM1 and PH2 JMF.  They are sampling daily from 1600-2400 at DSM1 and approximately 0800-1100 at PH2 JMF.

3.8. USGS Radio-Tracking.  USGS continued releasing and radio-tracking juveniles this reporting week.  They are tracking juveniles as part of the PH1 surface collector monitoring.

3.9. WDFW Fish Counting.  WDFW continued 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

3.10. WES/Battelle Hydroacoustic Monitoring.  WES and Battelle continued hydroacoustic monitoring of the PH1 surface collector this reporting week.

4.  FISHWAYS.  Project biologists inspected on 5, 8, and 9 June this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| 6/9 | PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | No action taken. The mechanical meter at the south end of the channel continues to work. |
| All week | FG 6-18 | Diffuser stuck partially open | Unknown | Will fix during winter maintenance period. |

4.2. Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder below the count station at Washington shore.  Temperatures are supplied by The Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/4 | 15.4 | 15.4 |
| 6/5 | 15.6 | 15.5 |
| 6/6 | 15.5 | 15.5 |
| 6/7 | 15.5 | 15.4 |
| 6/8 | 15.4 | 15.3 |
| 6/9 | 15.4 | 15.3 |
| 6/10 | 15.4 | 15.3 |

Fish Ladder Temperatures (C)

4.3. Sensor Calibration.  Water level sensors were checked for calibration on 7 June this reporting week.

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//