### CENWP-OP-B                                                                                                         19 June 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 24 of 2000, which covers the period 11 June through 17 June.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 188.5 to 259.4 kcfs.  Daily average spill ranged from 81.0 to 108.7 kcfs.  Daily average tailwater elevation for the project ranged from 16.1’ to 20.4’ msl.  River temperature ranged from 61 to 63° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

- Unit 12 remained out of service for stator rewedge and 4-year overhaul.

- The first powerhouse ice and trash sluiceway remained open this reporting week.  PH1 chain gates are being operated according to PSC testing requirements.  The second powerhouse sluiceway was operated according to Normandeau survival test requirements this reporting week.

- Second powerhouse fish units remained in service this reporting week with one exception.  Both fish units were out of service from 0442-1700 on 13 June for fishway inspection dives and semi annual maintenance.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1. AWS Closures.  FV3-9 was closed for cleaning from 1722-1732 on 14 June.

2.2. Dewaterings and Fish Salvage.  There was no dewaterings or fish salvage this reporting week.

2.3. STS Inspections.  Three units’ STS’s were inspected this reporting week and five that were not reported the previous week.  All screens were in good condition unless otherwise noted below.  See Table 1.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1 | 6/7 | 723 |  |
| 2 | 6/8 | 722 |  |
| 4 | 6/8 | 744 |  |
| 5 | 6/8 | NA |  |
| 6 | 6/12 | NA |  |
| 7 | 6/12 | 771 | Repaired small rip in B-slot screen. |
| 10 | 6/12 | NA |  |

Table 1.  STS Inspection Data.

- RESEARCH.

3.1. AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts following shad passage season.

3.2. AFC&MF Adult Sampling and Tagging.  The University of Idaho continued radio tagging and tracking adult chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult chinook and steelhead approximately 2 days per week for their age composition research.

3.3. Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington shore ladders this year.

3.4. Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers began releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for their survival testing this reporting week.  They will test through 18 June and resume a second set of tests in mid September.  This work is in support of corner collector research.

3.5. NMFS FGE and OPE Testing.  NMFS resumed FGE and OPE testing in unit 8B and 9B slots this reporting week. Testing was conducted nightly from approximately 1700-2330 on 12 through 16 June this reporting week.

3.6. NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

3.7. PH2 Bypass System Evaluation.  NMFS discontinued releasing PIT-tagged juvenile chinook and steelhead through the new bypass system this reporting week to determine if fish condition is adversely affected by system passage.  They will resume testing on 26 June by releasing 300 juveniles per week for the next 4 weeks to complete their testing.  USGS continued radio-tagging juveniles this reporting week for release in the DSM and tracking from the outfall release to Beacon Rock.

3.8. PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC continued 24-h/d PIT tag interrogation at DSM1 and PH2 JMF.  They are sampling daily from 1600-2400 at DSM1 and approximately 0800-1100 at PH2 JMF.

3.9. University of Idaho Lamprey Test Flume.  The University of Idaho began testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF this reporting week.  Testing began on 12 June and will continue through early August depending on lamprey availability.

3.10. USGS Radio-Tracking.  USGS continued releasing and radio-tracking juveniles this reporting week.  They are tracking juveniles as part of the PH1 surface collector monitoring.

3.11. WDFW Fish Counting.  WDFW continued 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

3.12. WES/Battelle Hydroacoustic Monitoring.  WES and Battelle continued hydroacoustic monitoring of the PH1 surface collector this reporting week.

4.  FISHWAYS.  Project biologists inspected on 13, 14, and 15 June this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser stuck partially open and should be closed. | Unknown | Will fix during winter maintenance period. |
| 6/14, 6/15 | PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | No action taken. |

4.2. Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder below the count station at Washington shore.  Temperatures are supplied by The Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/11 | 15.3 | 15.3 |
| 6/12 | 15.4 | 15.3 |
| 6/13 | 15.6 | 15.6 |
| 6/14 | 16.1 | 16.0 |
| 6/15 | 16.4 | 16.3 |
| 6/16 | 16.4 | 16.3 |
| 6/17 | 16.5 | 16.4 |

Fish Ladder Temperatures (C)

4.3. Sensor Calibration.  Water level sensors were checked for calibration on 13 June this reporting week.

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5.  OTHER.

5.1. Fishway Dives.  Divers inspected the PH1 collection channel, B-branch ladder entrance, Cascades Island ladder entrance, PH2 north and south monoliths, and PH2 collection channel on 12 and 13 June this reporting week.  All gratings and diffusers were in good condition.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//