### CENWP-OP-B                                                                                                         17 July 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 28 of 2000, which covers the period 10 July through 15 July.

- OPERATION SUMMARY.

1.1. Average daily river flow ranged from 180.5 to 199.6 kcfs.  Daily average spill ranged from 81.5 to 112.4 kcfs.  Average daily tailwater elevation for the project ranged from 15.6’ to 16.7’ msl.  Average daily forebay elevation for the project ranged from 73.4’ to 74.0’ msl.  River temperature ranged from 66 to 68° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

- Unit 12 remained out of service for stator rewedge and 4-year overhaul.  Unit 11 went out of service at 0230 and Units 13 and 14 at 0500 10 July until 1600 13 July for T11 annual maintenance.  Unit 3 was out of service several hours on 11, 12, 13 July for rehab testing.

- The first powerhouse ice and trash sluiceway remained open this reporting week.  PH1 chain gates are being operated according to PSC testing requirements.

- Second powerhouse fish unit 1 went out of service 2312 09 July until 0906 11 July for a transformer outage.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1. AWS Closures.  There were no AWS closures this reporting week.

2.2. Dewaterings and Fish Salvage.  There were no dewaterings or fish salvages this reporting week.

2.3. STS Inspections.  Eight STS’s were inspected this reporting week.  All screens were good unless otherwise stated.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 3 | 7/10 | 793 |  |
| 1 | 7/11 | 771 |  |
| 6 | 7/11 | 723 |  |
| 9 | 7/11 | 750 |  |
| 10 | 7/11 | 673 |  |
| 11 | 7/11 | 331 |  |
| 2 | 7/12 | 745 |  |
| 13 | 7/12 | 180 |  |

Table 1.  STS Inspection Data.

RESEARCH.

3.1. AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts following shad passage season.

3.2. AFC&MF Adult Sampling and Tagging.  The University of Idaho continued radio tagging and tracking adult chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult chinook and steelhead approximately 2 days per week for their age composition research.

3.3. Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington shore ladders this year.

3.4. Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers will resume releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for  survival testing in mid September.  This work is in support of corner collector research.

3.5. NMFS FGE and OPE Testing.  NMFS completed FGE and OPE testing in unit 8B and 9B slots this reporting week.

3.6. NMFS Lamprey Tagging and Radio Tracking.  NMFS continued trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

3.7. PH2 Bypass System Evaluation.  NMFS released PIT-tagged juvenile chinook through the new bypass system this reporting week to determine if fish condition is adversely affected by system passage.  They completed testing the end of this week.  USGS radio-tagged juveniles this reporting week for release in the DSM and for tracking from the outfall release to Beacon Rock.  They will complete their study on or around 24 July.

3.8. PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC continued 24-h/d PIT tag interrogation at DSM1 and PH2 JMF.  They are sampling daily from 1600-2400 at DSM1 and approximately 0800-1000 at PH2 JMF.

3.9. University of Idaho Lamprey Test Flume.  The University of Idaho continued testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF this reporting week.  Testing will continue through early August depending on lamprey availability.

3.10. USGS Radio-Tracking.  USGS continued releasing and radio-tracking juveniles this reporting week.  They are tracking juveniles as part of the PH1 surface collector monitoring.

3.11. WDFW Fish Counting.  WDFW continued 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

3.12. WES/Battelle Hydroacoustic Monitoring.  WES and Battelle continued hydroacoustic monitoring of the PH1 surface collector this reporting week.

4.  FISHWAYS.  Project biologists inspected on 10, 12 and 13 July this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | No action currently taken. |

4.2. Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder below the count station at Washington shore.  Temperatures are supplied by The Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 7/9 | 18.8 | 18.7 |
| 7/10 | 18.9 | 18.8 |
| 7/11 | 18.8 | 18.7 |
| 7/12 | 18.9 | 18.7 |
| 7/13 | 19.2 | 19.1 |
| 7/14 | 19.3 | 19.1 |
| 7/15 | 19.2 | 19.1 |

Fish Ladder Temperatures (C)

4.3. Sensor Calibration.  Water level sensors were checked for calibration on 14 July this reporting week.

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//