### CENWP-OP-B                                                                                                     07 August 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 31 of 2000, which covers the period 30 July through 05 August.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 142.3 to 190.3 kcfs.  Daily average spill ranged from 82.3 to 96.0 kcfs.  Average daily tailwater elevation for the project ranged from 13.4’ to 16.1’ msl.  Average daily forebay elevation for the project ranged from 73.4’ to 74.3’ msl.  River temperature ranged from 71 to 72° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 12 remained out of service for stator rewedge and 4-year overhaul.  Unit 3 remained out of service for turbine rehab.  Unit 4 went out of service 0724 31 July for it’s 100 day inspection.  Unit 6 went out of service at 0912 31 July for index testing.  Unit 8 was out of service from 1948 31 July until 0806 2 August due to a bad ESBS screen.  Unit 7 went out of service from 2130 3 August until 2154 4 August because of a thrust oil temperature alarm.

  - The first powerhouse ice and trash sluiceway remained open this reporting week.  As of 0715 2 August, PH1 chain gates are in normal operation.

  - Second powerhouse fish units are in service and operating normally this week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  FV 3-9 closed from 1402 until 1418 1 August and from 1455 until 1506 3 August for cleaning.

  - Dewatering and Fish Salvage.  Unit 4 draft tube- recovered 3 sturgeon and 1 lamprey.

  - STS Inspections.  No STS’s were inspected this reporting week.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts following shad passage season.

  - AFC&MF Adult Sampling and Tagging.  The University of Idaho continued radio tagging and tracking adult chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult chinook and steelhead approximately 2 days per week for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers will resume releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing in mid September.  This work is in support of corner collector research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS continued trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

  - PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC continued 24-h/d PIT tag interrogation at DSM1 and PH2 JMF.  They are sampling daily from 1600-2400 at DSM1 and approximately 0800-1000 at PH2 JMF.

  - University of Idaho Lamprey Test Flume.  The University of Idaho continued testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF this reporting week.  Testing will continue through early August depending on lamprey availability.

  - WDFW Fish Counting.  WDFW continued 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

4.  FISHWAYS.  Project biologists inspected on 01, 02 and 03 August this reporting week.

4.1

Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |

4.2  Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 7/30 | 21.3 | 21.2 |
| 7/31 | 21.6 | 21.4 |
| 8/01 | 21.5 | 21.4 |
| 8/02 | 21.5 | 21.3 |
| 8/03 | 21.3 | 21.1 |
| 8/04 | 21.4 | 21.3 |
| 8/05 | 21.7 | 21.6 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 03 August this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- Miscellaneous

  - FV1-1 continues to be operated in manual due to PH1 PCC sensors not staying in calibration.

  - FV5-3 and FV5-4 returned to auto 0931 31 July.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//