### CENWP-OP-B                                                                                               05 September 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 35 of 2000, which covers the period 27 August through 02 September.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 101.8 to 148.0 kcfs.  Average daily spill ranged from 0 to 86.1 kcfs.  Average daily tailwater elevation for the project ranged from 9.9’ to 13.4’ msl.  Average daily forebay elevation for the project ranged from 73.6’ to 74.3’ msl.  River temperature ranged from 68 to 69° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 12 remained out of service for stator re-wedge and 4-year overhaul.  Unit 3 remained out of service for turbine rehab.  Unit 4 remained out of service for its 100 day inspection.  Unit 6 remained out of service for index testing.

  - Fish Unit 2 went out of service from 1206 until 1512 29 August for start/stop and excitor repairs, and from 1254 until 1306 30 August due to an emergency trip of the DC breaker.  Fish Unit 1 went to standby from 1030 until 1106 31 August.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  No closures.

  - Dewatering and Fish Salvage.  29 August biologists recovered 6 sculpin, 1 crappie, and 2 catfish from Unit 12 tail logs.  30 August biologists recovered 4 sturgeon, and 16 sculpin from Unit 4 tail logs.  All fish were in good condition.

  - STS Inspections.  No STSs were inspected this reporting week.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts following shad passage season.

  - AFC&MF Adult Sampling and Tagging.  The University of Idaho continued radio tagging and tracking adult Chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult Chinook and steelhead approximately 2 days per week for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers resumed releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing.  This work is in support of corner collector research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS continued trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

  - PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC continued 24-h/d PIT tag interrogation at DSM1 and PH2 JMF.  They are sampling daily from 1600-2400 at DSM1 and 24 h/d at PH2 JMF.

  - University of Idaho Lamprey Test Flume.  The University of Idaho continued testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF this reporting week.

  - WDFW Fish Counting.  WDFW continued 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

  - WDFW/ODFW Fall Chinook counting.  WDFW and ODFW started counting Fall Chinook at the Bradford Island and Washington shore count stations during the day.

  - Yakima Nation.  Tribal members are fishing for northern pikeminnow from the powerhouses during the afternoon and evening hours.

4.  FISHWAYS.

  - Project biologists inspected on 28, 29, 30, and 31 August this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |

4.2 Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 8/27 | 20.2 | 20.1 |
| 8/28 | 20.1 | 20.0 |
| 8/29 | 20.0 | 19.8 |
| 8/30 | 20.0 | 19.9 |
| 8/31 | 19.9 | 19.8 |
| 9/01 | 19.8 | 19.7 |
| 9/02 | 19.7 | 19.6 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 30 August this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

5.1 Bonneville stopped spilling 2359 31 August.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//