### CENWP-OP-B                                                                                               18 September 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 37 of 2000, which covers the period 10 September through 16 September.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 91.4 to 140.9 kcfs.  Spill has been stopped.  Average daily tailwater elevation for the project ranged from 8.8’ to 13.6’ msl.  Average daily forebay elevation for the project ranged from 73.6’ to 75.5’ msl.  River temperature ranged from 67 to 68° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 12 returned to service at 1654 11 September.  Unit 3 remained out of service for turbine rehab.  Unit 4 remained out of service due to high bearing temperatures.  Unit 6 remained out of service for 100-day inspection.  Unit 5 went out of service from 0418 13 September until 1636 14 September for semi annual maintenance on Bank 5/6.  Unit 5 went out of service again on 15 September from 0500 until 1030 for dive work on Unit 6 trashrack.  The ice and trash sluiceway at powerhouse one was closed for Unit 6 dive work from 0500 until 1030 15 September.

  - The Project went to Powerhouse two unit priority at 1800 13 September.  Powerhouse one had priority due to work being performed on the Hood River Line.

  - Second powerhouse fish units were operated normally all week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  No AWS closures.

  - Dewatering and Fish Salvage. On 11 September biologists recovered 11 live sturgeon and 15 dead sturgeon from Unit 6 draft tube.

  - STS Inspections.  No STSs were inspected this reporting week.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 18” orifice inserts remain installed in the return ladder of the AFC&MF.  NMFS will resume testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 18” and 26” orifice inserts 13 September through 28 September.

  - AFC&MF Adult Sampling and Tagging.  The University of Idaho continues radio tagging and tracking adult Chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult Chinook and steelhead approximately 2 days per week for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers will resume releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing in late September.  This work is in support of corner collector research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

  - PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC finished 24-h/d PIT tag interrogation at DSM1 as of 0012 9 September.  They continue PIT tag interrogation and 24 h/d sampling at PH2 JMF.

  - University of Idaho Lamprey Test Flume.  The University of Idaho continues testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF.

  - WDFW Fish Counting.  WDFW continues 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

  - WDFW/ODFW Fall Chinook counting.  WDFW and ODFW are still counting Fall Chinook at the Bradford Island and Washington shore count stations during the day.

  - Yakima Nation.  Tribal members are fishing for northern pikeminnow from the powerhouses during the afternoon and evening hours.

4.  FISHWAYS.

  - Project biologists inspected on 11, 12, 13, and 14 September this reporting week.  NMFS inspected 15 September.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |
| 9/11, 9/13, 9/14 | A-branch staff gauge | Water level reading below 1.0’ +/- .1’ | Unknown | No action taken at this time. |
| 9/10-11 | FG3-3 | Diffuser valve wouldn’t open. | Mechanical failure | FG3-4 opened in manual. FG3-3 fixed on 9/11. |

4.2 Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 9/10 | 18.9 | 18.8 |
| 9/11 | 19.0 | 18.9 |
| 9/12 | 19.4 | 19.3 |
| 9/13 | 19.5 | 19.4 |
| 9/14 | 19.6 | 19.5 |
| 9/15 | 19.7 | 19.6 |
| 9/16 | 19.6 | 19.5 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 13 September this reporting week.  PH1 south tailwater sensor was .6’ low.  A work action request was turned in to the electricians.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - FV1-1 in manual from 1100 10 September until 1555 11 September.

  - FG 3-4 open and in manual from 0600 10 September until 1555 11 September due to mechanical failure of FG3-3.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//