### CENWP-OP-B                                                                                               25 September 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 38 of 2000, which covers the period 17 September through 23 September.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 104.2 to 167.6 kcfs.  Average daily tailwater elevation for the project ranged from 10.4’ to 14.4’ msl.  Average daily forebay elevation for the project ranged from 73.1’ to 75.1’ msl.  River temperature averaged 67° F all week.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 3 remained out of service for turbine rehab.  Unit 4 remained out of service due to high bearing temperatures.  Unit 6 remained out of service for 100-day inspection.  Units 7 and 8 went out of service for transformer bank maintenance 0448 20 September.   Unit 7 returned to service  1224 21 September.  Unit 8 returned to service 0906 22 September.

  - Second powerhouse fish units were operated normally all week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  No AWS closures.

  - Dewatering and Fish Salvage.  The AFF was dewatered on 20 September to accommodate NMFS pit-tag detector installation.  Fish were guided downstream and blocked from re-entering the dry sections.

  - STS Inspections.  Two STSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 9 | 9/19 | 707 | Video inspection |
| 10 | 9/19 | 734 | Video inspection |

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields were installed in the return ladder of the AFC&MF.  NMFS is testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 26” orifice inserts.

  - AFC&MF Adult Sampling and Tagging.  The University of Idaho continues radio tagging and tracking adult Chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult Chinook and steelhead approximately 2 days per week for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers will resume releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing 27 September.  This work is in support of corner collector research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

  - PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC finished 24-h/d PIT tag interrogation at DSM1.  They continue PIT tag interrogation and 24 h/d sampling at PH2 JMF.

  - University of Idaho Lamprey Test Flume.  The University of Idaho continues testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF.

  - WDFW Fish Counting.  WDFW continues 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

  - WDFW/ODFW Fall Chinook counting.  WDFW and ODFW are still counting Fall Chinook at the Bradford Island and Washington shore count stations during the day.

  - Yakima Nation.  Tribal members are fishing for northern pikeminnow from the powerhouses during the afternoon and evening hours.

4.  FISHWAYS.

  - Project biologists inspected on 18, 19, 20, 21, and 22 September this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |

  - Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 9/17 | 19.4 | 19.4 |
| 9/18 | 19.4 | 19.4 |
| 9/19 | 19.4 | 19.4 |
| 9/20 | 19.4 | 19.4 |
| 9/21 | 19.2 | 19.1 |
| 9/22 | 18.5 | 18.3 |
| 9/23 | 18.1 | 18.0 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 19 September this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//