### CENWP-OP-B                                                                                               10 October 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 40 of 2000, which covers the period 01 October through 07 October.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 105.1 to 133.2 kcfs.  Average daily tailwater elevation for the project ranged from 10.0’ to 11.8’ msl.  Average daily forebay elevation for the project ranged from 74.4’ to 75.3’ msl.  River temperature ranged between 62 and 65° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 3 remained out of service for turbine rehab.  Unit 13 remained out of service for four year overhaul.  Unit 6 remained out of service for 100-day inspection.  Unit 4 came back in service 1224 06 October.  Unit 2 went out of service from 0654 until 0812 03 October for a Hub oil leak.

  - Second powerhouse fish units were operated normally all week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  No AWS closures.

  - Dewatering and Fish Salvage.  No fish salvage this week.

  - STS Inspections.  No STS inspections this reporting week.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields were installed in the return ladder of the AFC&MF.  NMFS is testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 26” orifice inserts.

  - AFC&MF Adult Sampling and Tagging.  The University of Idaho continues radio tagging and tracking adult Chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult Chinook and steelhead approximately 2 days per week for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers will complete releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing on 6 October.  This work is in support of corner collector research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

  - PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC finished 24-h/d PIT tag interrogation at DSM1.  They continue PIT tag interrogation and 24 h/d sampling at PH2 JMF.

  - University of Idaho Lamprey Test Flume.  The University of Idaho continues testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF.

  - WDFW Fish Counting.  WDFW continues 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

  - WDFW/ODFW Fall Chinook counting.  WDFW and ODFW are still counting Fall Chinook at the Bradford Island and Washington shore count stations during the day.

  - Yakima Nation.  Tribal members have completed fishing for northern pikeminnow from the powerhouses.

4.  FISHWAYS.

  - Project biologists inspected on 02, 03, and 04 October this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |
| All week | Cascade Island bird wires. | One wire is down. | Rangers cut wire to remove entangled gull. | Will repair during winter maintenance period. |

  - Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 10/01 | 18.1 | 17.9 |
| 10/02 | 17.9 | 17.7 |
| 10/03 | 17.5 | 17.3 |
| 10/04 | 17.2 | 17.0 |
| 10/05 | 16.9 | 16.7 |
| 10/06 | 16.1 | 16.0 |
| 10/07 | 15.9 | 15.8 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 04 October this reporting week.  Weir gate 2, 64, and 65 were out of calibration.  A work action request was submitted.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//