### CENWP-OP-B                                                                                               7 November, 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 44 of 2000, which covers the period 29 October through 04 November.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 116.4 to 139.2 kcfs.  Average daily tailwater elevation for the project ranged from 11.2’ to 12.2’ msl.  Average daily forebay elevation for the project ranged from 74.2’ to 75.0’ msl.  River temperature ranged between 58 and 59° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

  - Unit 3 remained out of service for turbine rehab. Unit 2 went out of service 0700 30 October for it’s four year overhaul.  Unit 4 and Unit 6 became available for service 02 November.  Unit 13 remained out of service for four-year overhaul.

  - Second powerhouse fish units operated normally this reporting week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  No AWS closures for this reporting week.

  - Dewatering and Fish Salvage.  Five sturgeon were recovered from Unit 2 draft tube 31 October.  All were in good condition.

  - STS Inspections.  No STS’s were inspected this reporting week.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields were installed in the return ladder of the AFC&MF.  NMFS is testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 26” orifice inserts.

  - AFC&MF Adult Sampling and Tagging. WDFW plans to sample adult Chinook and steelhead approximately 7 days per week, throughout the winter, for their age composition research.

  - Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS completed trapping and tagging lamprey in the AFC&MF.  They are still tracking the fish back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.

4.  FISHWAYS.

  - Project biologists inspected on 30, and 31 October and 02 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH1 and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |
| All week | Cascade Island bird wires. | One wire is down. | Rangers cut wire to remove entangled gull. | Will repair during winter maintenance period. |
| All week | FG3-3, FG3-4 | FG3-3 fails to fully open. | Mechanical trouble. | Will repair during winter maintenance. FG3-4 is open to supplement FG3-3. |

  - Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 10/29 | 14.0 | 13.9 |
| 10/30 | 13.9 | 13.8 |
| 10/31 | 13.9 | 13.8 |
| 11/01 | 13.8 | 13.7 |
| 11/02 | 13.6 | 13.5 |
| 11/03 | 13.6 | 13.5 |
| 11/04 | 13.4 | 13.2 |

Fish Ladder Temperatures (C)

  - Sensor Calibration.  Water level sensors were checked for calibration on 02 November this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - Because of mechanical reasons, DSM2 screen cleaners are run once per shift (or more often depending on debris loads) to reduce wear and tear on the equipment.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//