### CENWP-OP-B                                                                                               02 January, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 52 of 2000, which covers the period 24 through 30 December.

- OPERATION SUMMARY.

  - Average daily river flow ranged from 138.8 to 140.2 kcfs.  Average daily tailwater elevation for the project ranged from 12.3’ to 13.2’ msl.  Average daily forebay elevation for the project ranged from 72.2’ to 74.0’ msl.  River temperature ranged between 44 and 45° F.  Spill bay 1, which supplies attraction flow for Cascades Island ladder, remained open 4”.  Spill bay 18, which supplies attraction flow for Bradford Island ladder, remains on seal.

  - Unit 3 remained out of service for turbine rehab.

  - Fish Unit 2 remains out of service for annual maintenance.  The ice and trash sluiceway is being used as the AWS back up.  Fish Unit 1 is being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  Bradford Island AWS remains closed while the ladder is dewatered.   FV6-9 was closed for cleaning from 0725 until 0800 26 December.

  - STS Inspections.  STS’s are pulled from all units.

  - Dewatering and Fish Salvage.  1 juvenile steelhead, 12 lamprey, 1 sucker, 1 whitefish, 1 catfish, and 2 sculpin were recovered from FG3-12.

3.   RESEARCH.

  - AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields remain installed in the return ladder of the AFC&MF.

  - AFC&MF Adult Sampling and Tagging. WDFW is sampling adult steelhead approximately 5 days per week, throughout the winter, for age composition research.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS completed trapping and tagging lamprey in the AFC&MF.  They are still tracking the fish back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues 24-h/d video counting at Washington shore.

4.  FISHWAYS.

  - Project biologists inspected on 26, 27, and 28 December this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |
| All week. | PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps per the FPP. | Unknown. | Investigating alternative method of measure. |
| All week | Cascade Island bird wires. | One wire is down. | Rangers cut wire to remove entangled gull. | Will repair during winter maintenance period. |
| All week | Chain Gate 7A | Gate was closed. Should be set to 72’ msl. | Riggers are unable to remove the bulkhead in the slot. | Waiting for the proper equipment to remove bulkhead. |
| 12/26, 12/27 | DSM 2 channel elevation | Water level was below 64.2’ | Low forebay levels | No action taken. |

  - Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Thermometers are currently undergoing calibration.

  - Sensor Calibration.  Water level sensors were checked for calibration on 27 December this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - Because of mechanical reasons, DSM2 screen cleaners are run once per shift (or more often depending on debris loads) to reduce wear and tear on the equipment.

  - Fish were salvaged from beneath diffuser FG3-12 gratings because it will be filled with concrete this winter maintenance season.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//