### CENWP-OP-B                                                                                               30 January, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 04 of 2001, which covers the period 21 through 27 January.

- OPERATION SUMMARY.

  - Daily river flow averaged 128.7 to 149.6 kcfs.  Daily project tailwater elevation averaged 11.7’ to 11.9’ msl.  Daily project forebay elevation averaged 72.7’ to 75.3’ msl.  River temperature averaged 44° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, were closed to conserve water.

  - Unit 3 remained out of service for turbine rehab.   Unit 10 was out of service at 0500 on 22 January until 0900 25 January for a check valve replacement.  Unit 2 went out of service at 1730 on 27 January to drain thrust bearing oil.  Units 11-18 were out of service for a few hours each as transducers were installed on the trashracks.

  - Fish Units 1 and 2 remain out of service for annual maintenance.

- ACTIVITIES.

  - AWS Closures.  FV6-9 remains out of service while the Washington Shore ladder is dewatered.  FV5-3, FV5-4, and FV5-9 were closed at 0659 on 23 January to facilitate the Cascades Island fishway dewatering.  FV1-1 was closed from 1040 until 1050 26 January for cleaning.

  - STS Inspections.  STSs are pulled from all units.

  - Dewatering and Fish Salvage.   On January 23rd biologists removed 18 adult steelhead, 1 juvenile steelhead, and about two dozen resident fish from the Cascades Island ladder and diffuser pools.  On January 25th, biologists removed 1 salmon and 6 sculpin from Cascades Island ladder entrance.  On January 26th, biologists removed 1 juvenile hatchery chinook from the Powerhouse Two south monolith.

3.   RESEARCH.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS completed trapping and tagging lamprey in the AFC&MF.  They are still tracking the fish back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues 24-h/d video counting at Bradford Island.

4.  FISHWAYS.

  - Project biologists inspected on 22, 24, and 25 January this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. | PH1 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | Waiting on new velocity meters. |
| All week | Cascade Island bird wires. | One wire is down. | Rangers cut wire to remove entangled gull. | Will repair during winter maintenance period. |
| All week | Cascades Island diffuser FG6-18. | Diffuser is stuck partially open and should be closed. | Unknown. | Will repair during winter maintenance period. |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Thermometers are currently undergoing calibration.

  - Sensor Calibration.  Water level sensors were checked for calibration on 24 January this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - Because of mechanical reasons, DSM2 screen cleaners are run once per shift (or more often depending on debris loads) to reduce wear and tear on the equipment.

  - Cascades Island went out of service on 23 January.

  - DSM1 remains out of service for maintenance.

  - The Ice and Trash Sluiceway at Powerhouse One closed at 0722 on 22 January to conserve water.  If temperatures drop below freezing, the ITS will be opened as needed.  All chaingates were closed as well.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//