### CENWP-OP-B                                                                                                                   12 March 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 10 of 2001, which covers the period 04 through 10 March.

- OPERATION SUMMARY.

  - Daily river flow averaged 116.8 to 129.8kcfs.  Daily spill averaged 0 to 12.0 kcfs.  Daily project tailwater elevation averaged 11.4’ to 11.5 msl.  Daily project forebay elevation averaged 73.3’ to 74.2’ msl.  River temperature averaged 44 to 45° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, were opened to 4” on 05 March.

  - Unit 3 remained out of service for turbine rehab.   Unit 15 remained out of service for gate slot modifications.  Units 11, 17, and 18 were out of service for a few hours each on 08 March for STS installation.

  - Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.  FV3-9 was closed from 1514 until 1543 on 05 March for cleaning.  FV4-3 and FV4-4 at B-branch and FV5-3, FV5-4, and FV5-9 are all operating in manual until the computer system is reinstalled.

  - STS Inspections.  No STS, ESBS, or VBS inspections this week.

  - Dewatering and Fish Salvage.   One juvenile Spring Chinook was recovered from the upstream sill of the new Navlock on 06 March.

3.   RESEARCH.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS completed trapping and tagging lamprey in the AFC&MF.  They are still tracking the fish back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues 24-h/d video counting at Bradford Island and the Washington Shore ladders.

  - NMFS Saltwater rearing.  The Saltwater rearing facility is under construction.  Collection is expected to start in April.

  - WDFW Winter Steelhead.  WDFW has resumed winter steelhead trapping and tagging at the AFC&MF.

  - PSMFC Adult PIT.  Installation is nearly complete at the Washington Shore ladder.  All in-water work is done, wiring of the office and the detector boxes remains.

  - PSMFC Smolt Monitoring Program.  Smolt monitoring began at DSM1 on 08 March.

4.  FISHWAYS.

  - Project biologists inspected on 05, 06, and 09 of March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. | PH1and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | Waiting on new velocity meters. |
| 3/5, 3/6 | PH1 chaingates | Gates 10C and 7A were closed. |  | Gates were set at full open and 72’, respectively, on 3/9. |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Thermometers are currently undergoing calibration.

  - Sensor Calibration.  Water level sensors were checked for calibration on 07 March this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - DSM2 returned to service at 0940 on 08 March.  Powerhouse Two switched to the priority powerhouse at 1215 on 09 March.

  - The Ice and Trash Sluiceway at the First Powerhouse returned to normal operation on 09 March per the Fish Passage Plan.

  - The Ice and Trash Chute at the Second Powerhouse was opened to elevation 68’ at 1140 on 09 March to assist passage of spring creek chinook.

  - Began the first of three 50kcfs spillway releases for Spring Creek fish.  Spill began 10 March at 1800 and ended at 0600 the next morning.

  - The hydrocannons for the Second Powerhouse outfalls were watered up at 1440 on 10 March.

  - Spring Creek released their chinook on 08 March.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//