### CENWP-OP-B                                                                                                                   19 March 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 11 of 2001, which covers the period 11 through 17 March.

- OPERATION SUMMARY.

  - Daily river flow averaged 117.7 to 135.7kcfs.  Daily spill averaged 0 to 23.6 kcfs.  Daily project tailwater elevation averaged 11.3’ to 12.2 msl.  Daily project forebay elevation averaged 73.6’ to 75.1’ msl.  River temperature averaged 45 to 46° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”

  - Unit 3 remained out of service for turbine rehab.   Unit 15 remained out of service for gate slot modifications.

  - Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the computer system is reinstalled.  Bradford Island exit closed for cleaning from 0804 until 0819 on 13 March.

  - STS Inspections.  No STS, ESBS, or VBS inspections this week.

  - Dewatering and Fish Salvage.   No dewatering or fish salvage activities this week.

3.   RESEARCH.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS still track the lamprey back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues 24-h/d video counting at Bradford Island and the Washington Shore ladders.

  - NMFS Saltwater rearing.  The Saltwater rearing facility is under construction.  Collection is expected to start in April.

  - WDFW Winter Steelhead.  WDFW finished winter steelhead trapping and tagging at the AFC&MF on 16 March.

  - PSMFC Adult PIT.  Installation is complete at the Washington Shore ladder.

  - PSMFC Smolt Monitoring Program.  Smolt monitoring began at DSM2 on 12 March.  Monitoring at DSM1 has ceased since Spring Creek has passed and DSM2 is up and running.

  - WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.

  - USGS Juvenile Radiotracking.  Antennas are installed on the STS’s.  Other antenna installations are expected to be completed next week.

4.  FISHWAYS.

  - Project biologists inspected on 12, 14, and 15 of March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. | PH1and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | Waiting on new velocity meters. |
| 3/12 | DSM1 water levels | Add in water not controlling properly. | Unknown |  |
| 3/14 | PH1 ITS | ITS closed. | USGS in the BRZ | Back to FPP criteria by afternoon. |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/1 | 4.0 | 4.0 |
| 3/2 | 4.1 | 4.1 |
| 3/3 | 4.3 | 4.2 |
| 3/4 | 4.4 | 4.3 |
| 3/5 | 4.5 | 4.4 |
| 3/6 | 4.6 | 4.6 |
| 3/7 | 4.8 | 4.7 |
| 3/8 | 5.0 | 5.0 |
| 3/9 | 5.3 | 5.2 |
| 3/10 | 5.4 | 5.4 |
| 3/11 | 5.5 | 5.5 |
| 3/12 | 5.7 | 5.7 |
| 3/13 | 5.7 | 5.7 |
| 3/14 | 5.9 | 5.9 |
| 3/15 | 6.0 | 6.0 |
| 3/16 | 6.0 | 5.9 |
| 3/17 | 5.9 | 5.9 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 15 March this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - The Ice and Trash Chute at the Second Powerhouse remains open to elevation 68’ to assist passage of spring creek chinook.

  - The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.

  - DSM2 Add in water was closed at 1549 on 13 March due to 12 smolts being impinged on the perf plate.  At 1625 11A-S, F2A-N, and F2B-N were opened to keep the channel elevation within criteria.  At 1315 on 14 March, NMFS requested the Add in be opened and that we keep track of the dead fish.  As of 15 March there have been 6 dead smolts found on the perf plate due to the Add in water being in operation.

  - The Powerhouse One Ice and Trash Sluiceway was closed from 0743 until 1640 on 12 March and from 0730 until 1240 on 13 March for dive work.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//