### CENWP-OP-B                                                                                                                                     04 April 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 13 of 2001, which covers the period 25 through 31 March.

- OPERATION SUMMARY.

  - Daily river flow averaged 122.5 to 147.1kcfs.  Daily spill averaged 0 kcfs.  Daily project tailwater elevation averaged 10.1’ to 13.2 msl.  Daily project forebay elevation averaged 74.2’ to 75.8’ msl.  River temperature averaged 48 to 49° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during the evening.  Bay 18 went on seal from about 0700 until 1700 on 26 through 29 March and Bay 1 went on seal from 0700 until 1730 on 30 March to help facilitate  research equipment installation dives at the spillway.

  - Unit 3 remained out of service for turbine rehab.   Unit 15 remained out of service for gate slot modifications.  Unit 11 went out of service from 0636 until 1336 on 29 March for research transducer replacement.

  - Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the computer system is reinstalled.  FV4-3 and FV4-4 were closed for the research equipment installation dive from 0500 until 1700 on 26 through 29 March.  FV5-3 and FV5-4 were closed for the research equipment installation dive from 0500 until 1700 on 30 March.

  - STS Inspections.  No STS, ESBS, or VBS inspections this week.

  - Dewatering and Fish Salvage.   No dewatering or fish salvage activities this week.

3.   RESEARCH.

  - NMFS Lamprey Tagging and Radio Tracking.  NMFS still track the lamprey back through the project in cooperation with the University of Idaho.

  - WDFW Fish Counting.  WDFW continues video counting at Bradford Island and the Washington Shore ladders.  Visual counts are scheduled to begin on 1 April.

  - NMFS Saltwater rearing. The Saltwater rearing facility is under construction.  Collection is expected to start in April.

  - PSMFC Adult PIT.  Installation is complete at the Washington Shore ladder.   Collection and tagging of fish is expected to begin April 10th.

  - PSMFC Smolt Monitoring Program.  Smolt monitoring continues at the monitoring facility.  Monitoring at DSM1 is scheduled to begin 15 April.

  - WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.

  - USGS Juvenile Radiotracking.  Antenna and cable installations are almost complete.  Radio tagging begins 1 April.  Tracking is expected to begin 15 April.

  - USGS Adult EMG tracking.  Underwater and aerial antennas are installed around Bradford Island and on the pier noses of the spillway.  Collection and tagging is expected to begin next week.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  Tagging will begin 1 April.

4.  FISHWAYS.

  - Project biologists inspected on 26, 28, and 29  March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. | PH1and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | Waiting on new velocity meters. |
| All week | B-Branch water levels. | No differential between channel and TW. | FV4-3, 4-4 closed for dive work on spillway. | Fishway back in service after 1700. |
| One day this week. | Bonneville Project | Biologists failed to check calibration. | Workload did not match staffing. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/25 | 7.2 | 7.2 |
| 3/26 | 7.4 | 7.3 |
| 3/27 | 7.4 | 7.4 |
| 3/28 | 7.4 | 7.4 |
| 3/29 | 7.6 | 7.6 |
| 3/30 | 7.7 | 7.7 |
| 3/31 | 7.9 | 7.9 |

  - Sensor Calibration.  Water level sensors were not checked for calibration by biologists this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.

  - As of 22 March, DSM2 orifices 11A-S, F2A-N, and F2B-N are kept open to keep the channel elevation within criteria while Unit 15 is out of service.

  - The Powerhouse One Ice and Trash Sluiceway was closed from 1500 until 1525 on 26 March to remove a log raft.

GENE L. LADOUCEUR

# Operations Project Manager

//S//