### CENWP-OP-B                                                                                                                                    09 April 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 14 of 2001, which covers the period 01 through 07 April.

- OPERATION SUMMARY.

  - Daily river flow averaged 111.3 to 133.8kcfs.  Daily spill averaged 0 kcfs.  Daily project tailwater elevation averaged 10.8’ to 12.5 msl.  Daily project forebay elevation averaged 75.2’ to 75.8’ msl.  River temperature averaged 49 to 50° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”.

  - Unit 3 remained out of service for turbine rehab.   Unit 15 remained out of service for gate slot modifications.

  - Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the computer system is reinstalled.   The Bradford Island fish exit was closed for cleaning from 1032 until 1055 on 05 April.

  - STS Inspections.  Using video, the STS and VBS screens can be inspected at the same time.  The following is a list of units whose screens were inspected this reporting week and the run time between inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 4/4 | 639 |  |
| 12 | 4/4 | 622 |  |
| 13 | 4/4 | 591 | Belt changed on C-slot |
| 14 | 4/4 | 552 |  |
| 16 | 4/9 | 662 |  |
| 17 | 4/5 | 651 |  |
| 18 | 4/5 | 672 |  |

Table 1.  STS Inspection Data.

  - Dewatering and Fish Salvage.   No dewatering or fish salvage activities this week.

3.   RESEARCH.

  - WDFW Fish Counting.  WDFW began 16-hour visual counts at Bradford Island and Washington Shore ladders on 01 April.

  - NMFS Saltwater rearing. The Saltwater rearing facility is under construction.  Fish collection is scheduled to begin in late April.

  - PSMFC Adult PIT.  Installation is complete at the Washington Shore ladder.   Collection and tagging of fish is expected to begin April 10th.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  Monitoring at DSM1 is scheduled to begin 9 April.

  - WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.

  - USGS Juvenile Radiotracking.  Antenna and cable installations are almost complete.  Radio tagging begins 1 April.  Tracking is expected to begin 15 April.

  - USGS Adult EMG tracking.  Underwater and aerial antennas are installed around Bradford Island and on the pier noses of the spillway.  Collection and tagging is expected to begin the second week of April.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One are operated according to UI request.  This will continue until 31 October.   Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of the season.

  - CRITFC/ODFW.  Tagging of Spring Chinook began 03 April.  They expect to tag on Tuesdays and Thursdays, with Friday as an extra day if needed.

4.  FISHWAYS.

  - Project biologists inspected on 03, 04, and 06 April this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. | PH1and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | Waiting on new velocity meters. |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/1 | 8.2 | 8.1 |
| 4/2 | 8.2 | 8.1 |
| 4/3 | 8.1 | 8.1 |
| 4/4 | 8.2 | 8.2 |
| 4/5 | 8.3 | 8.3 |
| 4/6 | 8.3 | 8.3 |
| 4/7 | 8.3 | 8.3 |

  - Sensor Calibration.  Water level sensors were checked on 06 April for calibration by biologists this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.

  - As of 22 March, DSM2 orifice 11A-S, F2A-N, and F2B-N are kept open to keep the channel elevation within criteria while Unit 15 is out of service.

  - DSM2 Add-in water remains in operation.  There was one dead smolt found on the perf plate on 03 April.

  - On 6 April, the fish counters voiced concerns about Chinook swarming through the ladders at various intervals.  There were reports of seals or sea lions in the tailraces of both powerhouses.  The Project did not take any action towards removing the natural predators from the river system.

GENE L. LADOUCEUR

# Operations Project Manager

//S//