### CENWP-OP-B                                                                                                                                   21 May 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 20 of 2001, which covers the period 13 through 19 May.

- OPERATION SUMMARY.

  - Daily river flow averaged 111.6 to 180.4kcfs.  Daily spill averaged 0 to 49.8 kcfs.  Daily project tailwater elevation averaged 10.7’ to 15.5’ msl.  Daily project forebay elevation averaged 74.6’ to 75.8’ msl.  River temperature averaged 57 to 58° F.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during daylight hours.  They are closed for water conservation between sunset and one hour before sunrise per BPA request.

  - Unit 3 remained out of service for turbine rehab.  Unit 15 is placed on standby each night after FGE testing.  Unit 17 went out of service for asbestos testing from 1048 until 1600 on 16 May.

  - Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is installed.

  - STS/VBS Inspections.  No STS/VBS inspections this reporting week.

  - Dewatering and Fish Salvage.   No fish salvage this week.

3.   RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.

  - OCFWRU Barging condition study.  Began 24 April.

  - PSMFC Adult PIT.  Chinook are collected and tagged on Wednesdays.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  Monitoring at DSM1 also continues.

  - WES Hydroacoustic Monitoring.  Monitoring began  20 April.

  - UI Barging condition study. They began sampling juvenile fish from transport barges on 22 April.

  - USGS Juvenile Radiotracking.  Radio tagging began 1 April.  Tracking in progress as well.

  - USGS Adult EMG tracking.  Fish release and tracking began 18 April.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One operated according to UI request until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September.

  - CRITFC/ODFW.  Tagging Spring Chinook and steelhead on Tuesdays and Thursdays, with Friday as an extra day if needed.

  - WDFW.  Tagging Spring Chinook with jaw tags in conjunction with their tooth-net studies.

4.  FISHWAYS.

  - Project biologists inspected on 14, and 15 May this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| All week. |  | Project Biologists did not complete a third inspection this week. | Illness. |  |
| All week. | PH1and PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. | New velocity meters to be installed during mid-season dives. |
| 5/15 | BI, A and B Branch, CI, Weir 37 and 38 | Water elevations low. | Not yet programmed for Shad Passage Season. | Electricians re-setting valves for Shad passage. |
| 5/15 | FG3-25 | Closed | Valves are in manual. | Operator opening valve. |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 5/13 | 13.6 | 13.6 |
| 5/14 | 13.6 | 13.4 |
| 5/15 | 13.4 | 13.3 |
| 5/16 | 13.3 | 13.2 |
| 5/17 | 13.2 | 13.0 |
| 5/18 | 13.4 | 13.2 |
| 5/19 | 13.7 | 13.5 |

  - Sensor Calibration.  Water level sensors were not checked for calibration this reporting week.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - 24 hour spill at 50 cfs began on 16 May at 1800.

  - All ladders set in Shad passage mode as of 0800 16 May.

  - DSM2 Add-in water remains in operation.  As of 08 May, Add-in flow has been shut off and a caution order tag hung on the valve.

  - 11A-S remains closed in manual, orifice lights are off.

  - The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.

  - Throughout the week there were reports of seals or sea lions in the tailraces of both powerhouses.  The Project did not take any action towards removing the natural predators from the river system.

GENE L. LADOUCEUR

# Operations Project Manager

//S//