### CENWP-OP-B                                                                                                                                  16 July 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 28 of 2001, which covers the period 08 through 14 July.

- OPERATION SUMMARY.

  - Daily river flow averaged 75.9 to 100.8kcfs.  Daily spill averaged 0.0 kcfs.  Daily project tailwater elevation averaged 7.3’ to 9.6’ msl.  Daily project forebay elevation averaged 74.2’ to 75.3’ msl.  River temperature averaged 65 to 67° F.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during daylight hours.

  - Unit 3 remained out of service for turbine rehab.  Unit 17 remains out of service for its four-year overhaul.  Unit 5 remains out of service for turbine rehab.  Unit 15 is on standby each night following FGE testing.  On 09 July; Units 9 and 10 were out of service for switching from 0248 until 0418, Unit 4 went out of service at 0424 for roof disconnect work and semi annual maintenance, Unit 11 went out of service from 0447 until 1854 for semi annual maintenance,  Unit 15 went out of service for VBS cleaning at 1242.  It remained OOS until FGE tests began that night.   Also on 9 July, Unit 10 went IN service as voltage support for 115KV system at 1248.  Unit 12 went out for semi annual maintenance from 0436 until 1812 on 10 July.  Unit 13 went out for semi annual maintenance from 0430 until 1800 on 11 July.  Unit 14 underwent semi annual maintenance from 0430 until 1718 on 12 July.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is installed.  FV3-9 was closed for cleaning from 0855 until 0905 on 09 July and from 1420 until 1440 on 12 July.

  - STS/VBS Inspections.  Two STSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 15 | 7/9 | 639 | Full of grass, cleaned and back in service. |
| 16 | 7/9 | 955 | First chance to inspect when not testing. |

Table 1.  STS/VBS Inspection Data.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage this reporting week.

- RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - OCFWRU Barging Condition.  Sampling to end 13 July.

  - PSMFC Adult PIT.  Looking at A and B branch and Cascade Island for noise interference.  Analyzing data collected at the Washington Shore ladder.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  24 h/d flat plate monitoring at DSM1.

  - WES Hydroacoustic Monitoring.  Monitoring began 20 April.  Should be complete by 15 July.  Equipment removal scheduled 30 July – 9 August.

  - UI Barging Condition and Separation By Code study.  Summer studies complete.  Will conduct fall chinook separation by code study 15 July through 17 September.

  - USGS Juvenile Radiotracking.  Radio tagging began 1 April.  Tracking is expected to be complete by 31 July.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One are being operated according to UI request until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September.

  - CRITFC/ODFW/WDFW Adult Trapping/Tagging.  Tagging Spring Chinook and steelhead on Tuesdays and Thursdays.

  - NMFS FGE/OPE.  Work began 23 April.  Spring testing ended ~ 23 May.  Summer tests began 11 June and should be complete by 13 July.

  - NMFS Lamprey.  Lamprey are tagged and tracked throughout the week.  Spillway ladder entrances manually operated at night for a .5’ differential.  NMFS is holding extra lamprey for the Warm Springs Tribe, and collecting fish for USGS.

  - YIN Northern Pikeminnow Dam Angling.  Fishing 25 June – 30 September.

4.  FISHWAYS.

  - Project biologists inspected every day (08-14 July) this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | PH2 is not yet working. | Not fully installed. | New velocity meter installed in channel, awaiting core drilling at PH2. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers are being designed, and will be installed during winter maintenance. | 20013904 |
| 7/10 | PH1 Collection channel velocity | Velocity meter registers less than 1.5 fps | Velocity meter in a slow area of the channel. | Velocity improves when OG/SG gates are open. FFU looking at the flow model, will suggest possible solutions. |  |
| All week | FG3-27 | Not in correct position for TW elevation. | FG3-27 is open to supplement FG3-28 | No action taken. |  |
| 7/8 | FG6-15 | Not in correct position for TW elevation. | Computer system not yet installed, all Cascade Island diffusers in manual | Operator corrected diffuser positioning |  |
| 7/12 | FG3-3 | Not in correct position for TW elevation. | Unknown | Corrected itself. |  |
| 7/12, 7/14 | Entrance gates at B-Branch and Cascade Island Ladders. | Not in correct position for TW elevation. | Computer system not yet installed, both ladders are operated in manual | Operators corrected gate positioning. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 7/8 | 18.7 | 18.7 |
| 7/9 | 19.1 | 19.1 |
| 7/10 | 19.4 | 19.4 |
| 7/11 | 19.5 | 19.5 |
| 7/12 | 19.8 | 19.7 |
| 7/13 | 19.9 | 19.8 |
| 7/14 | 19.8 | 19.7 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 12 July.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - 12 dead lamprey are floating in the Cascade Island ladder above the count station.  They got into the ladder before the perforated bulkhead was lowered on 31 May.

  - PH2 JBS injury/mortality issue: Juvenile mortality remains low at the JMF, according to PSMFC reports.  We plan no further action at this time.

  - Due to low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gates 2 and 64 are now maintaining a 1.5’ head differential.

  - FV1-1 will not control properly in auto.  It has been in manual since 13 July to maintain differential levels.

  - DSM2 Screen cleaner #2 broke at 0345 on 14 July.  It was fixed and back in auto by 1100 14 July.

  - PH1 Collection Channel velocity is very low when orifice block tests require orifices be closed.  We can overcome the problem when tailwater is greater than or equal to 10 ‘.  Tailwater is dropping lower than that now, so the channel is sometimes out of criteria.  We continue working with FFU and District Bios to find a solution.

  - Spill stopped at 0000 16 June.  All bays are on seal during evening hours.  Bays 1 and 18 will remain open 4” during daylight hours.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//