### CENWP-OP-B                                                                                                                               19 August, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 33 of 2001, which covers the period 12 through 18 August.

- OPERATION SUMMARY.

  - Daily river flow averaged 102.6 to 120.2kcfs.  Daily spill averaged 49.3 to 49.7kcfs.  Daily project tailwater elevation averaged 9.7’ to 11.0’ msl.  Daily project forebay elevation averaged 74.5’ to 75.3’ msl.  River temperature was 70° F all week.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”.  Night spill (2000 until 0100) of 45 kcfs began on 24 July.  24 hour spill of 50kcfs began at 1400 on 09 August and will continue until further notice.

  - Unit 3 remains out of service for turbine rehab.  Unit 17 remains out of service for its four-year overhaul.  Unit 5 remains out of service for turbine rehab.  Unit 9 went out of service from 2318 12 August until 0930 13 August and at 0442 on 14 August for roof disconnect work.  Unit 18 went out of service from 0236 until 0900 on 14 August due to high temperatures.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.    F2 was out of service from 1918 until 2236 on 15 August for trashrack cleaning.  A 4’ drawdown occurred about 1730.

- ACTIVITIES.

  - AWS Closures. FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is calibrated.   FV3-9 was closed for cleaning from 0726 until 0744 on 13 August, 1455 until 1508 on 14 August, and 1526 until 1535 on 16 August.

  - STS/VBS Inspections.  No STSs were video inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage this reporting week.

- RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - PSMFC Adult PIT.  Looking at A and B branch and Cascade Island for noise interference.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  All flat plate monitoring at DSM1 ended at 0800 on 02 August.

  - UI Barging Condition and Separation By Code study.  Fall Chinook separation by code study runs 15 July through 17 September.

  - USGS Juvenile Radiotracking.  Equipment removal in progress.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at the first powerhouse are being operated according to UI request until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September.   Researchers are adding ice to the anesthetic tank when water temperatures approach 70° F.

  - CRITFC/ODFW/WDFW Adult Trapping/Tagging.  Tagging Chinook and steelhead on Tuesdays and Thursdays.

  - NMFS Lamprey.  Lamprey are tagged and tracked throughout the week.  Spillway ladder entrances manually operated at night for a .5’ differential.  NMFS is collecting fish for USGS.

  - YIN Northern Pikeminnow Dam Angling.  Fishing 25 June – 30 September.

  - WDFW/ODFW Fall Chinook Counting.  Observations of the upriver fall Chinook began 14 August at both ladders.

4.  FISHWAYS.

  - Project biologists inspected 12 through 18 August this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers are being designed, and will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | No action taken. |  |
| 8/12-13, 8/15-18 | PH1 Collection channel velocity | Velocity meter registers less than 1.5 fps. | Water backs up in the channel during OG/SG tests. | Monitoring situation. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Wire grid to be replaced in Mid-April. |  |
| 8/15 | SG 34 | Not in correct position for testing. | Unknown | Operators closed gate manually. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 8/12 | 21.7 | 21.7 |
| 8/13 | 21.8 | 21.7 |
| 8/14 | 21.8 | 21.7 |
| 8/15 | 21.9 | 21.7 |
| 8/16 | 21.7 | 21.5 |
| 8/17 | 21.6 | 21.4 |
| 8/18 | 21.5 | 21.4 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 18 August.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - PH2 JBS injury/mortality issue: Juvenile mortality remains low at the JMF, according to PSMFC reports.

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - Closed OG/SG testing causes low differential at the north end of PH1 collection channel.  FV1-1 normally controls south channel differential, but is now reprogrammed to maintain north channel differential.  This change should also help maintain proper collection channel velocity during OG/SG tests.

  - The B-Branch and Cascade Island computers were installed.  Sensors still need calibrated and some parameters are not set quite right yet.  Until then, entrance gates and FV4-3, 4-4, 5-3, and 5-4 are operated in manual.

  - Large flocks of gulls have been congregating around the spillway.  Avian wires were not reinstalled after the dives since 24 hour spill went immediately into effect.

  - PH2 units were operated out of FPP priority 14-15 August for 3D Cam testing.  Units were operated within the 1% peak efficiency.  Testing went from 1100 until 1615 on 14 August and from 0800 until 1620 on 15 August.

  - FV3-9 was in manual control from 1845 17 August until 1110 18 August due to a float control alarm that wouldn’t clear.  The float control has been repaired.

  - PH2 Corner Collector exploratory drilling along the north side of Cascades Island began this week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//