### CENWP-OP-B                                                                                                                           10 September, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 36 of 2001, which covers the period 02 through 08 September.

- OPERATION SUMMARY.

  - Daily river flow averaged 76.0 to 92.9kcfs.   Daily project tailwater elevation averaged 6.9’ to 8.8’ msl.  Daily project forebay elevation averaged 75.4’ to 75.7’ msl.  River temperature averaged 67 to 70° F.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”.  This translates to a daily spill average of 2.3kcfs.

  - Unit 3 remains out of service for turbine rehab.  Unit 17 remains out of service for its four-year overhaul.  Unit 5 remains out of service for turbine rehab.  Units 7 and 8 went out of service from 1924 on 03 September until 1742 on 08 September for a bank7/8 bushing replacement.  Units 6 and 9 went out of service from 0400 on 04 September until 1612 on 08 September for roof disconnect work.  Units 1,2 and 10 went out of service roughly two hours each on 04 September for STS removal.  Unit 4 went out of service for STS removal on 05 September.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures. FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is calibrated.  FV3-9 and the Bradford Island exit were closed for cleaning from 0645 until 0704 on 04 September.  FV5-9 was closed for cleaning from 0710until 0733 on 04 September and from 1544 until 1554 on 05 September.  The Bradford Island exit was closed for cleaning from 1040 until 1058 on 05 September.

  - STS/VBS Inspections.  No STSs were video inspected this reporting week.

  - STS removal. PH1 STS were pulled 04-06 September per NMFS request.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage this reporting week.

- RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - PSMFC Adult PIT.  Tagging to begin again on 18 September at the AFF.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  All flat plate monitoring at DSM1 ended at 0800 on 02 August.  GBT exams concluded 30 August.

  - UI Barging Condition and Separation By Code study.  Fall Chinook separation by code study runs 15 July through 17 September.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at the first powerhouse are being operated according to UI request until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September.   Researchers are adding ice to the anesthetic tank when water temperatures approach 70° F.

  - CRITFC/ODFW/WDFW Adult Trapping/Tagging.  Tagging Chinook and steelhead on Tuesdays and Thursdays.

  - NMFS Lamprey.  Tagging complete.  Tracking continues.  Spillway ladder entrances manually operated at night for a .5’ differential.   377 lamprey were collected for USGS and 138 were collected for the Warm Springs Tribe.

  - YIN Northern Pikeminnow Dam Angling.  Fishing 25 June – 30 September.

  - WDFW/ODFW Fall Chinook Counting.  Observations of the fall Chinook continue.

4.  FISHWAYS.

  - Project biologists inspected 02 through 08 September this reporting week.   Salmon Corps volunteers inspected 06, 07, and 08 September.  NMFS inspected on 07 September.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers are designed, and will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | No action taken. |  |
| 9/5-8 | PH1 Collection channel velocity | Velocity meter registers less than 1.5 fps. | Water backs up in the channel during OG/SG tests. | Monitoring situation. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Wire grid to be replaced in Mid-April. |  |
| 9/3-4 | Weir 37 | Reading >1.0’ | Stillwell full of debris. | Stillwell cleaned. |  |
| 9/4 | UMT staff gauge | Reading >1.0’ | FV5-9 float not controlling properly. | Electricians corrected the problem. |  |
| 9/7 | BI Picket leads | Head greater than .3’ | A lot of debris. | Riggers cleaned the leads that afternoon |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 9/2 | 21.4 | 21.2 |
| 9/3 | 21.3 | 21.1 |
| 9/4 | 21.1 | 21.0 |
| 9/5 | 20.6 | 20.6 |
| 9/6 | 20.1 | 20.1 |
| 9/7 | 19.7 | 19.7 |
| 9/8 | 19.6 | 19.5 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 06 September.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - PH2 JBS injury/mortality issue: Juvenile mortality remains low at the JMF, according to PSMFC reports.

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - Closed OG/SG testing causes low differential at the north end of PH1 collection channel.  FV1-1 normally controls south channel differential, but is now reprogrammed to maintain north channel differential.  This change should also help maintain proper collection channel velocity during OG/SG tests.

  - The B-Branch and Cascade Island computers were installed.  Sensors need calibrated and some parameters need setting.  Until then, entrance gates and FV4-3, 4-4, 5-3, and 5-4 are operated in manual.

  - Spillway avian wires were removed 19 July to facilitate deflector work.

  - Salmon Corps continue volunteer work.

  - In an effort to keep A and B branch “depth over weir” in criteria, the depth at the Bradford Island staff gauge will be increased to 1.0’ +/- .1’.  The FPP calls for .9’ water over weir, but at that depth, A and B Branches were reading .1-.2’ too low.

  - Large numbers of fish have been passing via Washington Shore ladder.  Many of those are fish holding  below the AFF picket leads when the trap is in use.  Project Biologists are opening the leads for a few hours during the day to try to move fish through the project in a timely manner.  Picket leads were up from 1440 until 1500 on 06 September, from 0830 until 0900 and from 1450 until 1550 on 07 September.

  - The outfall hydrocannon was shut off at 0943 on 05 September.  We have not observed any bird activity.

  - The high water alarm in the AFF has been modified.  Fish splashed water on the alarm sensor as they entered the return ladder, setting off the alarm.  At the end of the tagging day, researchers, operators, or project biologists need to turn the alarm on.

  - The return ladder trashrack was cleaned on 06 September.  1 clipped Chinook, 1 clipped steelhead, and 1 unidentifiable were removed and discarded down the “glory hole”.  The fish were in progressed states of decay.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//