### CENWP-OP-B                                                                                                                           24 September, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 38 of 2001, which covers the period 16 through 22 September.

- OPERATION SUMMARY.

  - Daily river flow averaged 76.8 to 94.3kcfs.   Daily project tailwater elevation averaged 7.8’ to 8.3’ msl.  Daily project forebay elevation averaged 74.7’ to 75.5’ msl.  River temperature averaged 67 to 69° F.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”.  This translates to a daily spill average of 2.3kcfs.

  - Unit 3 remains out of service for turbine rehab.  Unit 17 remains out of service for its four-year overhaul.  Unit 5 remains out of service for turbine rehab.  Unit 6 returned to service at 1730 on 21 September.  Units 7 and 8 went out of service from 1600 until 1630 on 20 September for switching.  Units 4 and 10 returned to service at 1630 on 20 September.  Unit 9 returned to service at 1200 on 20 September.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

- ACTIVITIES.

  - AWS Closures. FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is calibrated.  FV3-9 was closed for cleaning from 1455 until 1510 on 17 September.  Bradford Island exit was closed for cleaning from 1600 until 1606 on 18 September.  FV5-9 was closed for cleaning from 1615 until 1626 on 18 September.

  - STS/VBS Inspections.  Four STSs were video inspected last reporting week and one this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 12 | 9/12 | 509 |  |
| 13 | 9/12 | 280 |  |
| 14 | 9/12 | 267 |  |
| 15 | 9/17 | 698 |  |
| 16 | 9/12 | 706 |  |

Table 1.  STS/VBS Inspection Data.

  - STS removal. PH1 STS remain OOS per NMFS request.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1 | 9/4 | 48 | Removed for season |
| 2 | 9/4 | 26 | Removed for season |
| 3 |  |  | Already out |
| 4 | 9/5 | 7 | Removed for season |
| 5 |  |  | Already out |
| 6 | 9/13 | 364 | Removed for season |
| 7 | 9/15 | 279 | Removed for season |
| 8 | 9/10 | ESBS | Removed for season |
| 9 | 9/4 | 205 | Removed for season |
| 10 | 9/4 | 312 | Removed for season |

Table 2.  STS Removal Data.

  - Dewatering and Fish Salvage.  Unit 6 draft tube was dewatered on 9/17, no fish were found.  Unit 6 taillogs were pulled on 9/20, no fish were found.

- RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - PSMFC Adult PIT.  Tagging to begin again on 18 September at the AFF.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  All flat plate monitoring at DSM1 ended at 0800 on 02 August.  GBT exams concluded 30 August.

  - UI Barging Condition and Separation By Code study.  Fall Chinook separation by code study to end this week.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at the first powerhouse are being operated according to UI request until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September.   Researchers are adding ice to the anesthetic tank when water temperatures approach 70° F.

  - CRITFC/ODFW/WDFW Adult Trapping/Tagging.  Tagging Chinook and steelhead on Tuesdays and Thursdays.

  - NMFS Lamprey.  Tagging complete.  Tracking continues.  Spillway ladder entrances manually operated at night for a .5’ differential.   377 lamprey were collected for USGS and 138 were collected for the Warm Springs Tribe.

  - YIN Northern Pikeminnow Dam Angling.  Fishing 25 June – 30 September.

  - WDFW/ODFW Fall Chinook Counting.  Observations of the fall Chinook continue.

4.  FISHWAYS.

  - Project biologists inspected 16 through 22 September this reporting week.   Salmon Corps volunteers inspected 16, 19, 20, 21,and 22 September.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers are designed, and will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | No action taken. |  |
| 9/18-22 | PH1 Collection channel velocity | Velocity meter registers less than 1.5 fps. | Water backs up in the channel during OG/SG tests. | Monitoring situation. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Wire grid to be replaced in Mid-April. |  |
| 9/20 | B-branch staff gauge | Reading <1.0’ | Unknown. A-branch in criteria. | No action taken. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 9/16 | 20.6 | 20.6 |
| 9/17 | 20.2 | 20.1 |
| 9/18 | 19.8 | 19.7 |
| 9/19 | 19.4 | 19.4 |
| 9/20 | 19.2 | 19.1 |
| 9/21 | 19.1 | 19.0 |
| 9/22 | 19.1 | 19.0 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 21 September.

  - Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- MISCELLANEOUS

  - PH2 JBS injury/mortality issue: Juvenile mortality remains low at the JMF, according to PSMFC reports.

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - Closed OG/SG testing causes low differential at the north end of PH1 collection channel.  FV1-1 normally controls south channel differential, but is now reprogrammed to maintain north channel differential.  This change should also help maintain proper collection channel velocity during OG/SG tests.

  - The B-Branch and Cascade Island computers were installed.  Sensors need calibrated and some parameters need setting.  Until then, entrance gates and FV4-3, 4-4, 5-3, and 5-4 are operated in manual.  Waiting on parts.

  - Salmon Corps continue volunteer work.

  - Large numbers of fish have been passing via Washington Shore ladder.  Many of those are fish holding below the AFF picket leads when the trap is in use.  On Thursday 13 September, NMFS requested lead #2 be open with lead #1.  On Sunday 16 September, fish numbers had dropped such that all picket leads could be down during sampling.

  - The outfall hydrocannon remains off.  We have not observed any bird activity.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//