### CENWP-OP-B                                                                                                                               22 October, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 42 of 2001, which covers the period 14 through 20 October.

- OPERATION SUMMARY.

  - Daily river flow averaged 79.7 to 119.0kcfs.   Daily project tailwater elevation averaged 7.4’ to 10.9’ msl.  Daily project forebay elevation averaged 71.8’ to 73.8’ msl.  River temperature averaged 60 to 62° F.

  - Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4”.  This translates to a daily spill average of 2.3kcfs.

  - Unit 3 remains out of service for turbine rehab.  Unit 5 remains out of service for turbine rehab.    Station service Unit 0 remains out of service.  Units 4 and 6 were out of service from 1230 until 1418 on 14 October due to a loss of preferred AC to the governor pumps.  Unit 15 went out of service at 0930 on 15 October for a 4-yr overhaul.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.    Fish units were out of service from 0800 until 1030 on 17 October for trashrack raking.

- ACTIVITIES.

  - AWS Closures. FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is calibrated.

The following dates and times are when the respective valves were closed for cleaning.

15 October, FV1-1from 1550 until 1640;

16 October, FV5-3, FV5-4 from 1230-1320, FV4-3, FV4-4 from 1320- 1605, FV1-1 from 1355- 1550;

17 October, FV6-9 from 1342-1405 and FV5-9 from 1405- 1430;

18 October, FV3-9 and FV1-1 from 1500-1545;

19 October, FV3-9 from 1525-1534, FV1-1 from 1542- 1555, and FV6-9 from 1602-1619;

20 October, FV3-9 from 1541-1557 and FV6-9 from 16-2 until 1618.

  - STS/VBS Inspections.  No STSs were video inspected this reporting week.

  - STS removal. PH1 STS remain OOS per NMFS request.

  - Dewatering and Fish Salvage.  On 16 October, project bios removed 23 sturgeon, 2 peamouth and 1 channel catfish from Unit 15 draft tube.

- RESEARCH.

  - WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - PSMFC Adult PIT.  Planning for construction at the Bradford Island and Cascade Island ladders continues.

  - PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the PH2 monitoring facility.

  - UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at the first powerhouse are being operated according to UI request until 31 October.  Tagging ended on 17 October.

  - CRITFC Adult Trapping/Tagging.  Tagging ended on 17 October.

  - ODFW/WDFW Adult Trapping/Tagging.  Tagging and trapping on Tuesdays.

4.  FISHWAYS.

  - Project biologists inspected 14-20 October this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers are designed, and will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | No action taken. |  |
| 10/14-18, 10/20 | PH1 Collection channel velocity | Velocity meter registers less than 1.5 fps. | Water backs up in the channel during OG/SG tests. | Monitoring situation. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Wire grid to be replaced in Mid-April. |  |
| 10/14, 10/19-20 | CI entrances | Not in correct position for TW elev. | Entrance gates are operated manually. | Operators adjusted gates. |  |
| 10/14-15 | F2B-S | Closed | Faulty proximity switch. | Operators put in local control and open. | 20012337 completed10/15 |
| 10/15-16 | BI fishways | Low water over weirs | Unknown | No action taken. |  |
| 10/15, 10/18, 10/20 | PH1 ITS | Closed | BRZ work | No action taken |  |
| 10/16, 10/19-20 | B-Branch entrances | Not in correct position for TW elev. | Entrance gates are operated manually. | Operators adjusted gates. |  |
| 10/16, 10/18 | DSM2 | Channel Elev. Low | Low forebay elevation. | Add-in opened. See misc. 5.9 |  |
| 10/16, 10-19-20 | Weir 37 | Reading >1.1’ | See misc. 5.5 | No action taken |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 10/14 | 15.9 | 15.9 |
| 10/15 | 15.7 | 15.6 |
| 10/16 | 15.7 | 15.4 |
| 10/17 | 15.6 | 15.4 |
| 10/18 | 15.5 | 15.4 |
| 10/19 | 15.4 | 15.3 |
| 10/20 | 15.4 | 15.2 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 19 October.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - PH2 JBS injury/mortality issue: Juvenile mortality remains low at the JMF, according to PSMFC reports.

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - Closed OG/SG testing causes low differential at the north end of PH1 collection channel.  FV1-1 normally controls south channel differential, but is now reprogrammed to maintain north channel differential.  This change should also help maintain proper collection channel velocity during OG/SG tests.

  - The B-Branch and Cascade Island computers were installed.  Sensors need calibrated and some parameters need setting.  Until then, entrance gates and FV4-3, 4-4, 5-3, and 5-4 are operated in manual.  Waiting on parts.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter which should correct the problem.

  - The outfall hydrocannon remains off.  We have not observed any bird activity.

  - Powerhouse One Ice and Trash Sluiceway was closed for work in the BRZ from 0752 until 1645 on 15 October, from 0930 until 1200 on 18 October and from 0555 until 0650 on 20 October.

  - At 0805 on 18 October, a hard forebay constraint requiring 71.5’- 72.5’ forebay elevation between the hours of 0630 and 1700 until 20 October, went into effect.

  - DSM2 Add-in water was opened and placed in automatic control at 2200 on 18 October.  The DSM channel was at low elevation due to the forebay constraint.  Six morts were found on the perf plate on 19 October.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//