### CENWP-OP-B                                                                                                                           04 November, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 44 of 2001, which covers the period 28 October through 03 November.

- OPERATION SUMMARY.

  - Daily river flow averaged 90.4 to 107.3kcfs.   Daily project tailwater elevation averaged 8.3’ to 9.6’ msl.  Daily project forebay elevation averaged 72.5’ to 75.2’ msl.  River temperature averaged 56 to 58° F.

  - Spill Bay 18 was closed at 1330 on 31 October for spill deflector work.  Bay 1 was opened 8” to provide additional adult attraction flow.  This translates into a daily spill average of 2.3kcfs.

  - Units 3 and 5 remain out of service for turbine rehab.  Station service Unit 0 remains out of service.  Unit 15 remains out of service for a 4-yr overhaul.  Unit 9 was out of service from 1318 until 1600 for governor repair work on 30 October.

  - Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.    F2 went to speed no load from 0945 until 1300 on 01 November for trashrack raking.

- ACTIVITIES.

  - AWS Closures.   The following dates and times are when the respective valves were closed for cleaning.

29 October- FV1-1 from 1510 until 1536, BI exit from 1547 until 1554;

01 November- FV1-1/ FV5-9/ FV6-9 from 1320 until 1550, FV3-9 from 1500 until 1550.

  - STS/VBS Inspections.  Two STSs were video inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 13 | 10/30 | 521 |  |
| 14 | 10/30 | 308 |  |

Table 1.  STS/VBS Inspection Data.

  - STS removal. PH1 STS remain OOS per NMFS request.

  - Dewatering and Fish Salvage.  No dewater or fish salvage activities to report.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video counts began 01 November at BI and WA. Shore.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

  - PSMFC Adult PIT.  Planning for construction at the Bradford Island and Cascade Island ladders continues.

  - PSMFC Smolt Monitoring Program.  Monitoring concluded at 0700 on 31 October at the JMF.

  - UI Adult Fallback. PCC entrance gates at PH1 went to auto on 01 November

  - WDFW Winter Steelhead tagging.  Tagging will begin 05 November.

4.  FISHWAYS.

  - Project biologists inspected 28-31 October and 01-03 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | See above. |  |
| 10/28-11/2 | PH1 PCC velocity | Velocity meter registers <1.5 fps. | Water backs up in the channel. | Monitoring situation. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Grid to be replaced in Mid-April. |  |
| 11/1 | CI entrances | Not in correct position for TW elev. | Entrance gates are operated manually. | Operators adjusted gates. |  |
| 11/1 | B-Branch entrances | Not in correct position for TW elev. | Entrance gates are operated manually. | Operators adjusted gates. |  |
| All week | Weir 37 | Reading >1.1’ | See misc. 5.4 | See misc. 5.4 |  |
| 10/31-11/3 | Spill bay 18 | On seal | Spillway deflector installation. | Spill bay 1 opened 8”. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 10/28 | 13.5 | 13.4 |
| 10/29 | 13.1 | 13.1 |
| 10/30 | 13.0 | 12.8 |
| 10/31 | 13.0 | 12.6 |
| 11/01 | 13.0 | 12.6 |
| 11/02 | 12.8 | 12.6 |
| 11/03 | 12.9 | 12.6 |

  - Sensor Calibration.  Water level sensors were checked for calibration on 03 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - The JMF was switched to bypass at 0710 on 31 October.

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.

  - One adult wild steelhead was found on the DSM2 grate near 11A-S on 27 October.  This should have been  included in last week’s report.

  - A forebay level restriction caused water levels in DSM2 to drop below 64.2’.  To compensate, the Add-in valve was opened from 0700 on 30 October until 0130 on 01 November.  Two live adult salmonids, 1 dead adult salmonid, and 6 dead juvenile shad were found behind the perf plate.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//