### CENWP-OP-B                                                                                                                          04 December, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 48 of 2001, which covers the period 25 November through 01 December.

- OPERATION SUMMARY.

  - Daily river flow averaged 115.4 to 146.2kcfs.  Daily project tailwater elevation averaged 11.6’ to 13.7’ msl.  Daily project forebay elevation averaged 74.7’ to 75.6’ msl.  River temperature averaged 50 to 51° F.

  - Spill Bay 18 was closed at 1330 on 31 October for spill deflector work.  Bay 1 was closed at 1048 on 30 November for spill deflector work.  This was coordinated in advance.  Spill averaged 0.0 to 2.6 kcfs.

  - Units 3 and 5 remain out of service for turbine rehab.  Unit 8 remains out of service for it’s five-year overhaul.  Unit 16 returned to service at 1612 on 29 November.  Unit 7 went out of service from 2242 on 26 November until 1524 on 27 November for removal of links on the 7/8 delta bus.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  Both fish units went to standby at 1136 on 01 December as the Washington Shore went to orifice flow in preparation for the 03 December dewatering.

- ACTIVITIES.

  - AWS Closures.   AWS valves were closed for trashrack cleaning as listed below.

26 November- FV3-7 from 1500 until 1520

FV3-9 from 1520 until 1540

FV5-9 from 1545 until 1600

FV6-9 from 1600 until 1620

  - STS/VBS Inspections.  One STS was video inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 16 | 11/27 | 741 |  |

Table 1.  STS/VBS Inspection Data.

  - STS removal. PH1 STS remain OOS per NMFS request.

  - Dewatering and Fish Salvage.  The AFF was dewatered on 29 November; 12 steelhead, 2 whitefish, 3 shad and 1 juvenile salmon were rescued and released to the Washington Shore Fish Ladder.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video counts ongoing at BI and WA Shore ladders.

  - NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.  Still have over 100 fish in the holding tanks.

  - PSMFC Adult PIT.  Planning for construction at the Bradford Island and Cascade Island ladders continues.

  - WDFW Winter Steelhead tagging.  Trapping has ceased while the AFF is dewatered.

4.  FISHWAYS.

  - Project biologists inspected 25 through 30 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 collection channel electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | FG3-28 | Out of service | Manually closed because of displaced diffuser valve cover. | New covers will be installed during winter maintenance. | 20013904 |
| All week | FG3-27 | Not in correct position for TW elev. | FG3-27 is open to replace FG3-28. | See above. |  |
| All week | Spill bay 18 | On seal | Spillway deflector installation. | Spill bay 1 opened 8”. |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Grid to be replaced in Mid-April. |  |
| All week | Weir 37 | Reading >1.1’ | See misc. 5.3 | See misc. 5.3 |  |
| All week | PH1 PCC velocity | Velocity meter registers <1.5 fps. | Water backs up in the channel. | Monitoring situation. |  |
| 11/26 | Bradford Island fishway staff gauges | Reading <1.0’ | Unknown | Operators informed |  |
| This week | Calibration checks | Not done | More work than available personnel | New help trained in calibration |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 11/25 | 9.9 | 9.7 |
| 11/26 | 9.9 | 9.8 |
| 11/27 | 9.7 | 9.7 |
| 11/28 | 9.3 | 9.1 |
| 11/29 | 9.0 | 8.8 |
| 11/30 | 8.9 | 8.8 |

  - Sensor Calibration.  Water level sensors were not checked for calibration this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.

  - PH2 Ice and trash chute was open from 1330 until 1350 on 26 November.

  - PH1 ITS was closed from 1349 until 1610 for forebay work on 27 November.

  - To aid spawning Chum below Bonneville, a tailwater restriction of 11.4’ to 11.6’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - WDFW biologists assisted with the AFF dewatering on 29 November.  The AFF will remain down until 07 December.

  - The riggers had FV5-3 and FV5-4 closed from 1218 until 1420 on 30 November while they set the bulkheads between Cascade Island and Spill Bay 1.  This was coordinated as part of the spillway deflector installation contract.

  - Washington Shore fishway went to orifice flow at 1136 on 01 December in preparation for the 03 December dewatering.  The Washington Shore ladder will be dewatered from the UMT to tailwater during 3-7 December due to Adult PIT tag antenna problems.  All ladder dewaterings have been postponed by one week.

  - Eight dead juvenile shad were found on the DSM2 grating around Units 11 and 12 this week.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//