### CENWP-OP-B                                                                                                                          17 December, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 50 of 2001, which covers the period 09 through 15 December.

- OPERATION SUMMARY.

  - Daily river flow averaged 104.7 to 132.6kcfs.  Daily project tailwater elevation averaged 11.7’ to 14.2’ msl.  Daily project forebay elevation averaged 73.8’ to 75.5’ msl.  River temperature averaged 48 to 49° F.

  - Spill Bay 18 and 1 are closed for spill deflector work.

  - Units 3 and 5 remain out of service for turbine rehab.  Unit 8 remains out of service for it’s five-year overhaul.  Unit 0 remains out of service.  Unit 6 went out of service from 0354 until 0424 and from 1624 until 1636 for switching work.  Unit 7 went out of service from 0354 until 1636 on 10 December for delta bus work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  F1 remains out of service for winter maintenance.    The ice and trash chute is being used to augment entrance attraction flow at PH2.

- ACTIVITIES.

  - AWS Closures. Bradford Island AWS valves closed for winter maintenance.

  - STS/VBS Inspections.  No STS inspections this reporting week.

  - Dewatering and Fish Salvage.  Bradford Island fishway was dewatered at 0724 on 10 December, B-branch entrance was dewatered on 11 December.  Biologists removed 16 wild adult steelhead; 26 hatchery adult steelhead; 1 clipped adult Chinook; 1 clipped jack Chinook; 1 lamprey; and 1 unclipped adult Coho from the Junction Pool.  9 hatchery adult steelhead and 1 five foot sturgeon were removed from A and B branch diffuser pools and entrance pool.  Approximately 12 juvenile salmonids and many resident fish were also removed.  All fish were released upstream.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video count ongoing the WA Shore ladder.

  - NMFS Saltwater rearing. All surviving fish were sacrificed on 10 December.  Saltwater removal and general clean up continue.

  - PSMFC Adult PIT.  Installation underway at Bradford Island ladder.

  - WDFW Winter Steelhead tagging.  Trapping continues four or five days a week.

4.  FISHWAYS.

  - Project biologists inspected 11 through 14 December this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 electronic velocity meter. | The velocity meter is not yet working. | Not fully installed. | Working on final installation. |  |
| All week | Spill bay 1 and 18 | On seal | Spillway deflector installation. |  |  |
| All week | Main Dam Avian wires | Wires removed. | Spillway deflector installation. | Grid to be replaced in Mid-April. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are not recorded during winter maintenance season.

  - Sensor Calibration.  Water level sensors were checked for calibration on 13 December this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Due to low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.

  - The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.

  - To aid spawning Chum below Bonneville, a tailwater restriction of 11.4’ to 11.6’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - PSMFC and University of Idaho biologists assisted with the Bradford Island fish ladder dewatering on 10 December.  See section 2.3.

  - DSM2 screen cleaner #1 was in manual from 1800 on 09 December until 1130 on 11 December because of  a faulty limit switch.

  - PH2 ice and trash chute supplementing  AWS water starting at 1100 on 12 December.  It was not used earlier as district engineers were inspecting the inside of the chute on 11 December.

  - Unit 18 and Fish Unit 2 raked for trash at 1312 and 1400, respectively, on 15 December.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//